NT Netcard Detection: Architecture and Interfaces

NT Networking

December 10, 1992

David Hovel

Revision 0.5

# 1.	Basic Approach

## 1.1.	Overview

This document describes the plans for network interface card detection in NT Beta 2 and beyond.  While the approach outlined here is simplistic, it should cover a very large majority of the real-world customer cases.

The components of the approach fall into several categories:

·	Initial Setup Detection

·	Configuration Querying

·	Configuration Verification

·	Interrupt Level Validation

The first three of these functions are supported by DLLs associated with the netcards themselves; the final function is contained within the SETUP/NCPA code.

## 1.2.	Architecture

### 1.2.1.	Detection Driver and DLL

The hardware interaction architecture is described in a later section of this document (see "Driver Interface Specification").  Briefly, a driver will be resident in NT which allows user programs to perform a narrow range of physical I/O and physical memory operations.

This driver will export its capabilities to the NT Win32 subsystem through a DLL.  This DLL can also verify that interface resources such as IRQs are available for use.  The support driver is named NETDTECT.SYS; its companion DLL is  NETDTECT.DLL.

The network installation process installs the driver as an optional service under the net "NetDetect".

### 1.2.2.	Netcard Interface DLLs

Each netcard, whether supported directly by Microsoft or by its manufacturer, will have a companion DLL which knows how to instruct the support driver's DLL to interact with the netcard.  Thus, the installer can interact directly with a netcard even though its proper network driver has not been loaded.

The netcard-level DLL interface will be constructed so that many different netcards can be handled by a single DLL.  All netcards supported for NT version 1 will be contained within a single DLL called MSNCDET.DLL.

A later chapter in this document defines the entry points for netcard detection DLLs.

## 1.3.	Usage

### 1.3.1.	Installation

The process called Initial Setup Detection only occurs once, during primary system installation.  It attempts to locate one instance of a known netcard.  Once this goal is achieved, the process ends.  This is the primary goal of the NT Beta 2 netcard detection software.

The results of this operation determine the subsequent recommendations made to the user.

It is important that the user interface allow users to bypass initial detection, since there is no way to entirely eliminate the possibility that such efforts may cause the computer to halt or fail, particularly on ISA-based machines.

### 1.3.2.	Post-Installation

Whenever a netcard is installed, either during main installation or OEM installation, the other detection processes are used.

Configuration Querying is done to try to present the user with the current card settings.

Configuration Verification is done to try to guarantee that the card can accept the user's choices.

Resource Validation is done to try to guarantee that the IRQ and other critical interface parameter choices do not conflict with other devices in the same computer.

## 1.4.	OEM Interfaces

### 1.4.1.	Extensibility

The netcard detection capabilities described in this document are intended to be extended by OEMs and ISVs.  An API interface is described which supports one or more netcards from one or more DLLs.

Microsoft will supply the basic driver  and DLL used to interrogate the hardware, along with the DLLs necessary to support its family of directly supported network cards.

However, the complete list of network card detection DLLs used by the Network Control Panel Applet and the NT SETUP program is extracted from the Registry at run time.   All information from all DLLs is merged together with information from the INF files found in the SYSTEM32 directory; the cumulative results are then used to perform auto-detection and provide user installation choices.

There are two separate times during which an OEM might want to add new network cards into the system ensemble.  First, as part of the "primary" system; that is, directly on the NT release media (CD-ROM, diskette) or on a pre-installed network share point.

Second, an OEM might want such support for netcards which are added to a running installation or which are supplied on add-on diskettes during installation.

### 1.4.2.	Primary Installation: OEMHOOK.INF

OEM netcard providers need to be able to add their information into the NT Registry before any netcard detection is attempted.  To facilitate this, the primary network installation file, NTLANMAN.INF, always calls (through the Win32 SETUP.EXE "Shell" command) a file called OEMHOOK.INF.

No errors are are reported if this file is not found.  It can perform any operations the OEM deems necessary.  The normal action would be to add the name of a DLL to the list of netcard detection DLLs.  For further details, see the section entitled "Multiple DLL Support" later in this document.

Use of OEMHOOK.INF allows an OEM to provide auto-detection for its netcards during primary system installation.

### 1.4.3.	Secondary Installation

Netcards can also be added after the system has been completely installed, or during a later phase of installation (in "custom" mode only) through the use of the Network Control Panel Applet.

In these cases, the supporting DLLs are not used for auto-detection;  their primary purpose is providing the installation INF file with the proper netcard parameter definitions and available value ranges.

# 2.	Initial Setup Detection

Initial Setup Detection is a process of iterating through the "known"  netcards until a clear indication of physical presence is given by the DLL.

This process is performed by the NTLANMAN.INF file during main installation.  Its purpose is the locate the "primary" netcard which the user will use for domain joining and  server initialization operations.

During "Custom" installations, the user will be given the choice of bypassing this detection scan.

Once a successful candidate has been located, the process will stop.  The user will then be given the choice of using the discovered card or choosing another from among the known card types.

Once the scan completes and the user has choosen a card type, the remainder of the functions of Network SETUP continue with little change.

# 3.	Secondary Netcard Installation

## 3.1.	Overview

The ability to install and interact with a netcard is governed by the presence of the INF and DLL files.  These files will be present for all the cards currently supported by Microsoft.

For OEM installations, the necessary INF and DLL files will be copied from the retailer's disk to the system directory before any other operations take place.

## 3.2.	Configuration Querying

Once a netcard type is chosen, the appropriate DLL will be selected and called in order to obtain the current strapped and software-selectable netcard settings.  These settings will minimally include:

·	I/O port address

·	IRQ number

·	Memory window status (disabled or address)

This DLL entrypoint also returns a "confidence factor"; that is, a number which indicates how reliably the software can detect the configuration.  If this number is low, the configuration dialog will be presented regardless of mode.  If this value is 100 (indicating complete certainty), the dialog will not be presented unless a system resource conflict is detected or the user is running in Custom mode.

### 3.2.1.	Querying During Express Mode

If the netcard configuration query is successful, SETUP will proceed immediately with the installation of the other network components.

If the query is unsuccessful, SETUP will present the netcard configuration dialog for the card in question.

### 3.2.2.	Querying During Custom Mode

If the netcard configuration query is successful, SETUP will fill the netcard configuration dialog with the known settings and present the dialog.

If the query is unsucessful, the dialog will be presented with the standard factory settings chosen.

### 3.2.3.	Querying During OEM Installation

Behavior during OEM installation is the same as for installation in Custom mode; the user is given full opportunity to change settings.

## 3.3.	Configuration Verification

Another DLL entry point allows SETUP to check that the entered settings are valid.  This will always be done, even if the user has not changed any of the settings reported by querying the configuration.

## 3.4.	Resource Allocation and Conflict

Netcards, like other hardware devices, require dedicated hardware resources for their operation.  On the Intel x86 architecture, these can include interrupt levels (IRQs), I/O addresses and ROM-space buffer addresses.

The netcard detection driver, NETDTECT.SYS, is used to "claim" and "free" resources at the NT kernel level.  If a resource claim succeeds, then the parameter setting is considered to be "permitted" by the current state of the system.

# 4.	DLL Entry Point Requirements

This section describes the exported functions available from any netcard detection support DLL.  Special parameters are used to allow a single DLL to support a large number of distinct netcard types.

In addition, there are some routines which are unique to the Microsoft netcard detetion support DLL (MSNCDET.DLL).  In addition to providing basic support for all the types of netcards directly supported by Windows NT, it exports functionality aimed at monitoring available system resources such as IRQ levels.

An OEM's SETUP INF file would typically contain the name of the DLL which can interrogate the netcard(s) it supports, and the string tokens representing those netcard types (see following text).

## 4.1.	Identify

This function returns information about the netcards supported by the DLL being addressed.

The function prototype is:

LONG NcDetectIdentify ( LONG lIndex, 
				   WCHAR * pwchBuffer, 
				   LONG cwchBuffSize ) ;

NcDetectIdentify() simply accepts a numeric index and responds with the appropriate NU:L-terminated string.  The return value is 0 if no error occured and the appropriate WINERROR.H value otherwise. The following indices are defined:

·	index 0 returns the manufacturer's identification for the DLL itself;

·	index 1 returns a date and version string (TBD);

·	netcard information indices start at 1000 in blocks of 100 values; for example, 1000 returns the internal short alphabetic token string representing the first supported netcard; 1001 returns the displayable name of the netcard (language-sensitive); 1002 returns the name of the manufacturer.and so on.  Index 1100 thus returns the alphabetic token for the second netcard type supported.  These values will be fully defined in the appendix (TBD).

Thus, the caller can simply start at 1000, incrementing by 100 and enumerate all the card types supported by a given DLL.  When the value ERROR_NO_MORE_ITEMS is returned, the enumeration is at an end.

If the buffer is too small, ERROR_INSUFFICIENT_BUFFER is returned.  This error is consisent throughout all netcard detection functions which return wide character data into a buffer.

The modulo-1000 values are refered to in prototypes as lNetcardId in the following sections.

One of the most important netcard parameters returned by NcDetectIdentify() is called the "search order", which has a value of 1003.  The result of this call is a numeric parameter, ranging in value from 0 to 1000, and is used by the user interface to sort its attempts to detect netcards.  The lower the number, the sooner the UI logic will attempt to detect that card.

Any netcard which does not specify a search order will be given a default value of 500.

The purpose of ordering the netcard detection search is to prevent certain unavoidable kinds of hardware interlock which can occur if tests are made in the wrong order.

## 4.2.	Query Configuration Parameters

The function prototype is:

LONG NcDetectQueryMask (  LONG lNetcardId,
					WCHAR * pwchBuffer,
					LONG cwchBuffSize ) ;

This function returns a buffer of UNICODE strings, each of which delimited by a UNICODE NUL.  The end of the buffer is indicated by an additional NUL (a zero-length string).

The strings are grouped into sets of three.  Each groups elements are:

first, the internal name of the parameter; this name must not contain special characters or blanks;

second, a character zero (0), one (1) or two (2);  a value of zero indicates that the parameter is not settable through software, a value of one indicates that it can be set through software, and a value of two indicates that it cannot be set through software and is not normally displayed to users;

third, a number from 0 to 100 indicating the confidence that the detection DLL has with regards to being able to detect the setting of the given parameter; 100 implies complete confidence.

For example (using the string (nul) to represent a binary zero character:

IRQ(nul)1(nul)70(nul) 
IOADDR(nul)0(nul)0(nul)
MEMADDR(nul)0(nul)0(nul)
MEMADDRLENGTH(nul)2(nul)100(nul)
(nul)

This buffer defines four parameter names: IRQ, IOADDR, MEMADDR and MEMADDRLENGTH.  Pointers to these strings will be used as parameters to other functions.

## 4.3.	Find First/Next Card by Type

This pair of routines is used to iterate over the instances of a particular type of netcard.   The prototype is:

LONG NcDetectFirstNext ( LONG lNetcardId,
					INTERFACE_TYPE itBusType,
					LONG lBusNumber,
					BOOL fFirst,
					PVOID * ppvToken,
					LONG * lConfidence );

The lNetcardId parameter contains the netcard index obtained from NcDetectIdentify(); it will always be a multiple 100 starting with 1000.

The fFirst parameter will be TRUE (1) to start the iteration and FALSE (0) otherwise.

The lBusNumber parameter indicates the bus being searched by the upper level detection algorithm.  This number will start at zero (0) and can continue through the number of busses found by NTDETECT.COM and stored into the Registry in the Hardware "hive".

The itBusType parameter contains an enumerated value which describes the type of bus on which the card may be found.  Again, this information is derived from the Registry by the upper level application.

The ppvToken parameter is a pointer to a PVOID which will contain a token to be used to obtain a handle for the card in question.

The lConfidence parameter will contain a value from 0 to 100 indicating how confident the underlying routine is about its ability to distinguish the presence of this instance of this card type.

## 4.4.	Open a Handle

The function prototype is:

LONG NcDetectOpenHandle (  PVOID pvToken,	
					PVOID * ppvHandle ) ;

This function is used to obtain a handle to a particular netcard whose existence has been reported by the detection DLL interface.

The ppvToken parameter must have been returned from a call to NcDetectFirstNext().

The ppvHandle parameter points to the receiving area for the opaque handle to be created by the detection DLL.

## 4.5.	Create a Handle (by Force)

The function prototype is:

LONG NcDetectCreateHandle (  LONG lNetcardId,
					INTERFACE_TYPE itBusType,
					  LONG lBusNumber,
					  PVOID * ppvHandle ) ;

This function is used to coerce the DLL into providing a handle for a netcard which does not appear to be present.  This is necessary when the user overrides detection and continues to configure a network card.

The lNetcardId parameter must contain a valid netcard type number.

For data on the lBusNumber and itBusType and parameters, see NcDetectFirstNext().

The ppvHandle parameter points to the receiving area for the resulting opaque token.

## 4.6.	Close Handle

The function prototype is:

LONG NcDetectCloseHandle ( PVOID * ppvHandle );

This function releases a handle returned by NcDetectOpenHandle().

## 4.7.	Query Configuration

The function prototype is:

LONG NcDetectQueryCfg ( PVOID pvHandle,
				   WCHAR * pwchBuffer,
				   LONG cwchBuffSize ) ;

This function is used to check the current configuration of the card.  The handle parameter must have been returned by a previous call to NcDetectOpenHandle().  The configuration parameter values are returned in a buffer as pairs of strings, each delimited by a UNICODE NUL character. For example:

IRQ(nul)0x310(nul)IOADDR(nul)0x368(nul)MEMADDR(nul)0xD8000(nul)(nul)

The values occur in pairs, where the value item name occurs first and the currently detected setting occurs second.  This format implies that the names of certain common parameters are clearly defined by this interface.  The currently defined names are:

IRQ
IOADDR
IOADDRLENGTH
MEMADDR
MEMADDRLENGTH
TRANSCEIVER

Others will probably be defined later.  A DLL and its INF file or installer program can use any other values for reporting and setting configuration information; however, all such private (non-Microsoft defined) values must begin with a '$' (dollar sign) character.

Standard 'C' language numeric literal conventions are used for all numbers. Numbers are assumed to be decimal unless prefixed with "0x". All string values are case-insensitive.  No ordering of return parameters can be assumed.

If no value is detectable for a particular parameter, it should not appear in the result buffer.

## 4.8.	Verify Configuration

The function prototype is:

LONG NcDetectVerifyCfg ( PVOID pvHandle,
				    WCHAR * pwchBuffer ) ;

This function uses a handle from a previous call to NcDetectOpenHandle() and supplies a buffer of configuration information formatted as in NcDetectQueryCfg().

The function returns NO_ERROR (0) if the configuration matches values which are either set on the card or are settable during driver initialization.  An error should be returned if critical (i.e., non-defaultable) parameters are missing from the buffer.

## 4.9.	Query Parameter Ranging

The function prototype is:

LONG NcDetectParamRange (  LONG lNetcardId,
					WCHAR * pwchParam,
					LONG * plValues,
					LONG * plValueCount ) ;

This routine accepts a netcard identity number and a pointer to the name of a parameter. It returns its results into an array of  LONGs (plValues) and a highest subscript for the given parameter.  On entry, the plValueCount parameter indicates the capacity in units of LONGs (i.e., buffer size / 4) of the buffer; on exit, the routine sets it to the highest value used plus one (limit of resulting plValues array).  For example, the UB/EOTP card's IRQ parameter might return {2.3,5,7.12}.

## 4.10.	Query Displayable Parameter Name

The function prototype is:

LONG NcDetectParameterName (  
		WCHAR * pwchParam,
		WCHAR * pwchBuffer, 							LONG cwchBufferSize) ;

This routine accepts the name of a parameter and returns a language-sensitive displayable name.

If the function returns ERROR_INVALID_PARAMETER, the upper-level components will be responsible for providing a displayable name.

## 4.11.	Resource Claiming Functions (MSNCDET.DLL only)

The functions listed in this seciton are only available from the Microsoft netcard detection DLL, MSNCDET.DLL.

These functions report the availablilty of and govern the allocation of bus- and hardware platform-related resources, such as interrupt levels and I/O space addresses.

### 4.11.1.	Claim a Resource

The function prototype is:

LONG NcDetectResourceClaim ( INTERFACE_TYPE ifType,
					LONG lBusNumber,
					LONG lType
					LONG lValue, 
					LONG lRange,
					LONG lFlags,
					BOOL fClaim ) ;

This function allows the caller to claim certain resources as "in-use".  This is necessary to verify that netcard parameters are actually available for use in the current system configuration.

The ifType parameter indicates the type of bus (ISA, EISA, etc.).  The lBusNumber parameter indicates which bus instance is being addressed.  The lType parameter indicates which system-level resource is being claimed.  The allowable values are:

NETDTECT_IRQ_RESOURCE	1	interrupt level
NETDTECT_MEMORY_RESOURCE	2	memory buffer addr range
NETDTECT_PORT_RESOURCE	3	i/o space addr range
NETDTECT_DMA_RESOURCE	4	DMA channel

The lValue parameter indicates the value of the resource type being claimed.  The lRange parameter applies only to certain parameters (such as memory windows) and gives the size in address units of the area being claimed.  As an example, only the lValue parameter would be meaningful for an IRQ parameter; the lRange parameter would be ignored.   Both values would be required for an IOADDR (port) parameter.

The lFlags parameter applies only to IRQ resources; allowable values are:

CM_RESOURCE_INTERRUPT_LEVEL_SENSITIVE	0
CM_RESOURCE_INTERRUPT_LATCHED			1

The fClaim parameter is used to allow the caller to check whether a resource can be claimed without actually claiming it.  If TRUE (1)  the resource is claimed; if FALSE (0), the resource is not claimed; the result is the same in either case.

To clarify, MSNCDET.DLL will keep a record of resources claimed by the process to which it is attached. When asked to claim or report resources later, MSNCDET will consider newly claimed resources, and report accordingly.  In other words, the information is used to augment the data extract from the Configuration Registry Hardware hive.

### 4.11.2.	Free a Resource

The function prototype is:

LONG NcDetectResourceFree ( INTERFACE_TYPE ifType,
					LONG lBusNumber,
					LONG lType,
					LONG lValue, 
					LONG lRange
					LONG lFlags ) ;

This function allows the caller to release resources previously claimed through the use of NcDetectResourceClaim().  The name of the parameter (or resource) and its values must exactly match a previously claimed value or ERROR_INVALID_PARAMETER will be returned.

# 5.	NCPA Exported Auxiliary Functions

Readers of this section are assumed to be familiar with the nature and syntax of the SETUP INF language.

## 5.1.	Purpose of the Auxiliary Functions

The executable file SETUP.EXE is an interpreter of the INF language, and, as such, it requires that all information passing back and forth between it and its extension DLLs be in textual form.  The Network Control Panel Applet has a set of auxiliary routines which provide such translation.  Currently, these NCPA "SETUP wrapper" routines are used to provide access to functions which are too complex to write in the SETUP INF language.

Several new wrappers will be added to encapsulate the functionality described in this document.  These new functions will be divided into two classes: those which are mere wrappers, and those which encompass algorithms to utilize the functions defined in this document.

## 5.2.	Multiple DLL Handling

One of the major purposes of these auxiliary functions is to handle the issue of dynamically installed netcard detection DLLs.

When a new netcard detection DLL is installed in the system, the installing software (INF file, executable, etc.) must add the name of the new DLL to the list contained in the REG_MULTI_SZ Registry value at:

\Registry\Machine\System\Setup

NetcardDlls REG_MULTI_SZ = MSNCDET.DLL

The DLL MSNCDET.DLL should always be first in this list.

All the DLLs in this list will automatically be loaded and queried about available netcards when the system is being installed.

## 5.3.	Detection Wrappers

These SETUP wrappers generally return a SETUP INF list of the form:

{ <error code>, arg, ... }

As in most symbolic languages, the parameters may be numbers, strings, atoms or other lists.   If an error occurs, SETUP will return only the error code.

Examples will be given later of the proper method for calling these functions.  It must be noted that the format used internally by SETUP for storage of nested lists is quite idiosyncratic (to put it mildly).  For this and other reasons, details of the implementation are not discussed in this section; instead, the empahsis is placed on the proper methods of access and usage.

Each of these wrapper functions is accessed through the NCPA.CPL export CPlSetup().  The standard form for such a call from an INF file is:

LibraryProcedure <ResultVarName> $(NcpaCplHandleVar) 		CPlSetup $(!STF_HWND) <FuncToBeRun> [other args]

Any additional arguments follow immediately behind the (case-insensitive) name of the function to be run.

The resulting list is stored into the <ResultVarName>. The first element of the list is the error code associated with the call or zero if no error occured.  The remaining elements of the list depend upon the function invoked.

If an error occurs, the variable FLibraryErrCtl is checked; if its value is 1, the error is returned to the calling INF; if it is undefined or not equal to one, SETUP displays the standard "DLL export call error" message; this should generally be avoided, since it can be quite cryptic to the casual user, and it allows retrying of the operation which almost always continues to fail.

### 5.3.1.	DTSTART

This function must be called by the SETUP INF file to initiate detection operations.  Multiple calls are ignored.   The format is:

DTSTART

In other words, only the function name is required.

The basic purpose of this function is to bind to all known netcard detection DLLs and create the data structures necessary to properly access and iterate over the DLLs.

### 5.3.2.	DTEND

This function should be called by the SETUP INF file to terminate detection operations.  It will be called automatically when SETUP unbinds from NCPA.CPL.  The format is:

DTEND

### 5.3.3.	DTRESET

This function causes all information about detected cards to be discarded, and the internal iteration control to be reset.  In other words, the state of the detection code after DTRESET is the same as after DTSTART The format is:

DTRESET

### 5.3.4.	DTCARD

This function attempts to detect the next card on the next bus in the iteration cycle.  The format is:

DTCARD

The result, if successful (error code == 0) is a list in the following form:

{ <error code>,
  OPTIONNAME,
  <card index>,
  <numeric card type code>,
  <detection confidence level>,
  <bus interface type>,
  <bus number>
}

The <card index> returned is a simple integer which the INF file must use to refer to this netcard in all future operations

### 5.3.5.	DTPARAMS

This function returns parameter information for a particular netcard option. The format is:

DTPARAMS <netcard option name>

The result is a list of lists describing the netcard parameters.  The basic results has the form:

{ <error code>
  <parameter description list>
}

where the <parameter description list> has the form:

{  {<parameter 1 name>,<setting control>,<confidence>,
	{v1, v2, v3, ...} },
   {<parameter 2 name>,<setting control>,<confidence>,
	{v1, v2, v3, ...} },
   ....
}

<Setting control> is an integer which determines whether the option is settable through software; <confidence> is an integer from 0 to 100 .  Each of the <vn> values is a legal setting for the parameter in question.

### 5.3.6.	DTQUERY

This function takes the index of a previously detected netcard and returns the currently detecable configuration parameters.

### 5.3.7.	DTVERIFY

This function takes a set of configuration parameters for a detected netcard and returns an indication of whether or not they are legal.

### 5.3.8.	DTSTRING

This function takes an option name and an integer and returns the corresponding string from the owning DLL.

### 5.3.9.	DTPNAME

This function takes an option name and a parameter name and returns the external (displayable) version of the name.

## 5.4.	NCPARAM.INF

Thie file NCPARAM.INF is similar to UTILITY.INF in that it provides a standard set of "callable" INF sections, usable throught the Win32 SETUP "Shell" command.

These INF sections understand how to interface to the detection wrapper functions documented above.

The details of the interaction between a netcard INF and NCPARAM.INF are too complex to document here; the DDK will contain the fully commented versions of UTILITY.INF, NCPARAM.INF and one or more ISA netcard INFs.  These should be studied carefully to understand the usage of NCPARAM.INF.

# 6.	Driver Interface Specification

## 6.1.	Overview

The model for network card detection is that card manufacturers will provide detection DLLs which are called in turn by the setup program during auto-detection and netcard installation.  (The discussion which follows is primarily directed at ISA machines, since the ISA bus does not have any configuration information associated with it.)

To allow a detection DLL access to the card ports and memory space, a simple NT kernel driver (NETDTECT.SYS) has been written which is accessed via IOCTLs submitted by the DLL. The driver provides four kinds of functions: port access, memory access, interrupt trapping and resource checking and claiming.

The detection DLL uses the port and memory access calls to attempt to locate cards of the type it recognizes. It will scan possible I/O port locations for a known signature or other characteristic to detect the card. It will then try to verify the card type by accessing the I/O ports or mapped memory to see if the card responds in a known way.

The detection DLL should do only what is needed to verify the card, and should err on the side of not detecting the card rather than perform dangerous operations. The assumption is that card detection may hang the system, and that this should be avoided if at all possible. In the case where detection hangs the system, the user will be instructed to not perform auto-detect and instead must specify the card explicitly.

Note that the convention is that one new card will be detected at a time. Previous cards will have registered their resources with the I/O system, and this information will be used to prevent access to ports or mapped memory that is in use by another driver.

A special routine is provided for cards where the driver cannot detect which interrupt they are using. The detection DLL tells the driver to trap all the interrupts that the card is known to support and which are unallocated in the current configuration. It then modifies the ports and mapped memory in such a way as to cause the card to generate an interrupt. A routine is provided for the detection DLL to determine which interrupts occured.

The interrupt trapping scheme may cause problems if the network card is jumpered to an interrupt which is in use by another device. The driver for the other device will see an unexpected interrupt. However, drivers on NT should deal with this. If not, the user will have been warned that the system may hang, as mentioned above.

The format of the data returned by the detection DLL is not defined in this portion of the specification.

## 6.2.	Functions Provided by the Driver

The routines are presented here in standard C calling convention.

### 6.2.1.	Resource Checking and Claiming

NTSTATUS
DetectCheckPortUsage(
	IN INTERFACE_TYPE InterfaceType,
	IN ULONG BusNumber,
	IN ULONG Port,
	IN ULONG Length
	);

NTSTATUS
DetectCheckMemoryUsage(
	IN INTERFACE_TYPE InterfaceType,
	IN ULONG BusNumber,
	IN ULONG BaseAddress,
	IN ULONG Length
	);

Thes functions will check if a range of ports/memory are in use by some other device in the system.  If  the ports/memory is available the return value will be STATUS_SUCCESS, otherwise it will be an error.

NTSTATUS
DetectClaimResource(
	IN ULONG NumberOfResources,
	IN PVOID Data
	);
typedef struct _NETDTECT_RESOURCE {
	INTERFACE_TYPE InterfaceType;
	ULONG BusNumber;
	ULONG Type;
	ULONG Value;
	ULONG Length;
	ULONG Flags;
} NETDTECT_RESOURCE, *PNETDTECT_RESOURCE;

//
// Resource types
//
#define NETDTECT_IRQ_RESOURCE
#define NETDTECT_MEMORY_RESOURCE
#define NETDTECT_PORT_RESOURCE
#define NETDTECT_DMA_RESOURCE

//
// Flags for claiming the IRQ resource
//
#define NETDTECT_IRQ_RESOURCE_LEVEL_SENSITIVE
#define NETDTECT_IRQ_RESOURCE_LATCHED

This function is used to claim a set of resources.  Data must point to an array of NETDTECT_RESOURCE structures, the length of the array is indicated via NumberOfResources.

### 6.2.2.	Port Access

These calls are used to access ports on the card, i.e. 0x300.

NTSTATUS
DetectReadPortUchar(
    IN INTERFACE_TYPE InterfaceType,
    IN ULONG BusNumber,
    IN ULONG Port,
   OUT PUCHAR Value 
    );

NTSTATUS
DetectWritePortUchar(
    IN INTERFACE_TYPE InterfaceType,
    IN ULONG BusNumber,
    IN ULONG Port,
    IN UCHAR Value,
    );

NTSTATUS
DetectReadPortUshort(
    IN INTERFACE_TYPE InterfaceType,
    IN ULONG BusNumber,
    IN ULONG Port,
    OUT PUSHORT Value,
    );

NTSTATUS
DetectWritePortUshort(
    IN INTERFACE_TYPE InterfaceType,
    IN ULONG BusNumber,
    IN ULONG Port,
    IN USHORT Value,
    );

NTSTATUS
DetectReadPortUlong(
    IN INTERFACE_TYPE InterfaceType,
    IN ULONG BusNumber,
    IN ULONG Port,
    OUT PULONG Value,
    );

NTSTATUS
DetectWritePortUlong(
   IN ULONG BusNumber,
    IN ULONG Port
   IN ULONG Value,
   );

These functions read or write data from I/O ports on the card.The return code indicates whether the data could be read or written successfully.   A detection DLL should check that a port is available by using DetectCheckPortUsage() before calling any of these functions.

### 6.2.3.	Memory Access

These functions are used to access card mapped memory, i.e. 0xCC000.

NTSTATUS
DetectReadMappedMemory(
    IN INTERFACE_TYPE InterfaceType,
    IN ULONG BusNumber,
    IN ULONG BaseAddress,
    IN ULONG Length,
    OUT PVOID Data
    );

NTSTATUS
DetectWriteMappedMemory(
    IN INTERFACE_TYPE InterfaceType,
    IN ULONG BaseAddress.
    IN ULONG Length,
    IN PVOID Data
    );

These functions read from or write to mapped memory. Data is a buffer provided by the user. The return code indicates if the memory could be read or written successfully. A detection DLL should verify that the memory is available by calling DetectCheckMemoryUsage() before using any of these functions.

### 6.2.4.	Interrupt Trapping

These function are used to determine which interrupt a card is using, for cards where this information cannot be read by any other means.

NTSTATUS
SetupInterruptTrap(
    IN INTERFACE_TYPE InterfaceType,
    IN ULONG BusNumber,
    OUT PHANDLE TrapHandle,
    IN UCHAR InterruptList[],
    IN ULONG InterruptListLength
    );

NTSTATUS
QueryInterruptTrap(
    IN HANDLE TrapHandle,
    IN OUT UCHAR InterruptList[]
    );

NTSTATUS
RemoveInterruptTrap(
    IN HANDLE TrapHandle
    );

SetupInterruptTrap takes a provided array of interrupt numbers. The driver will attempt to register interrupt handlers on each of the interrupts specified. A subset of these will typically succeed, based on what other devices are in the system. It will fail immediately if they are all in use.

After calling this routine, the detection DLL will cause the card to generate an interrupt. QueryInterruptTrap is then called to determine which trapped interrupts occurred. Each entry in InterruptList will be filled in as follows:

0 - no interrupt occured
1 - one interrupt occured
2 - more than one interrupt occured
3 - the interrupt is in use by another device

It is up to the detection DLL to wait an appropriate time for the interrupt to happen. The detection DLL should also perform the appropriate operations to dismiss the interrupt on the card. QueryInterruptTrap may be called multiple times and each time will return the results since the last call.

When the DLL no longer needs to trap interrupts, it calls RemoveInterruptTrap to remove the previously setup handlers.

## 6.3.	Access to the Driver

The DLL NETDTECT.DLL exports a user-level interface handles the above calls and converts them into the appropriate IOCTLs. This will be callable by any of the detection DLLs.

### 6.3.1.	DetectCheckPortUsage

NTSTATUS
DetectCheckPortUsage(
    IN  INTERFACE_TYPE InterfaceType,    
    IN  ULONG BusNumber,
    IN  ULONG Port,
    IN  ULONG Length
   );

### 6.3.2.	DetectReadPortUchar

NTSTATUS
DetectReadPortUchar(
    IN  INTERFACE_TYPE InterfaceType,
    IN  ULONG BusNumber,
    IN  ULONG Port,
    OUT PUCHAR Value
    );

### 6.3.3.	DetectReadPortUshort

NTSTATUS
DetectReadPortUshort(
    IN  INTERFACE_TYPE InterfaceType,
    IN  ULONG BusNumber,
    IN  ULONG Port,
    OUT PUSHORT Value
    );

### 6.3.4.	DetectReadPortUlong

NTSTATUS
DetectReadPortUlong(
    IN  INTERFACE_TYPE InterfaceType,
    IN  ULONG BusNumber,
    IN  ULONG Port,
    OUT PULONG Value
    );

### 6.3.5.	DetectWritePortUchar

NTSTATUS
DetectWritePortUchar(
    IN  INTERFACE_TYPE InterfaceType,
    IN  ULONG BusNumber,
    IN  ULONG Port,
    IN  UCHAR Value
    );

### 6.3.6.	DetectWritePortUshort

NTSTATUS
DetectWritePortUshort(
    IN  INTERFACE_TYPE InterfaceType,
    IN  ULONG BusNumber,
    IN  ULONG Port,
    IN  USHORT Value
    );

### 6.3.7.	DetectWritePortUlong

NTSTATUS
DetectWritePortUlong(
    IN  INTERFACE_TYPE InterfaceType,
    IN  ULONG BusNumber,
    IN  ULONG Port,
    IN  ULONG Value
    );

### 6.3.8.	DetectCheckMemoryUsage

NTSTATUS
DetectCheckMemoryUsage(
    IN  INTERFACE_TYPE InterfaceType,
    IN  ULONG BusNumber,
    IN  ULONG BaseAddress,
    IN  ULONG Length
    );

### 6.3.9.	DetectReadMappedMemory

NTSTATUS
DetectReadMappedMemory(
    IN  INTERFACE_TYPE InterfaceType,
    IN  ULONG BusNumber,
    IN  ULONG BaseAddress,
    IN  ULONG Length,
    OUT PVOID Data
    );

### 6.3.10.	DetectWriteMappedMemory

NTSTATUS
DetectWriteMappedMemory(
    IN  INTERFACE_TYPE InterfaceType,
    IN  ULONG BusNumber,
    IN  ULONG BaseAddress,
    IN  ULONG Length,
    IN  PVOID Data
    );

### 6.3.11.	DetectSetInterruptTrap

NTSTATUS
DetectSetInterruptTrap(
    IN  INTERFACE_TYPE InterfaceType,
    IN  ULONG BusNumber,
    OUT PHANDLE TrapHandle,
    IN  UCHAR InterruptList[],
    IN  ULONG InterruptListLength
   );

### 6.3.12.	DetectQueryInterruptTrap

NTSTATUS
DetectQueryInterruptTrap(
    IN  HANDLE TrapHandle,
    OUT UCHAR InterruptList[],
    IN  ULONG InterruptListLength
    );

### 6.3.13.	DetectRemoveInterruptTrap

NTSTATUS
DetectRemoveInterruptTrap(
    IN  HANDLE TrapHandle
    );

### 6.3.14.	DetectClaimResource

NTSTATUS
DetectClaimResource(
    IN  ULONG NumberOfResources,
    IN  PVOID Data
    );

# 7.	Appendix:  Netcard Information String Indices

This appendix contains the definitions and meanings of the strings which can be returned from the NcDetectIdentity() function in a netcard detection DLL.

| # Index | # Description | # Format |
| --- | --- | --- |

| # nn00 | # Option string for netcard in INF | # Unicode string; no imbedded delimeters |
| --- | --- | --- |

| # nn01 | # Displayable name of netcard | # Unicode string |
| --- | --- | --- |

| # nn02 | # Displayable name of Manufacturer | # Unicode string |
| --- | --- | --- |

| # nn03 | # Search order | # Unicode decimal numeric string |
| --- | --- | --- |

# 8.	Appendix: Known Netcard Configuration Parameter Strings:

This section lists the currently known textual configuration parameter strings, their meanings and their possibly ranges and values.  This list cannot be definitive, since vendors are free to create new parameters as needed.

| Parameter Name | Description | Possible Values and Range |
| --- | --- | --- |

| IRQ | Interrupt Request Level |  |
| --- | --- | --- |

| IOADDR | I/O Space address | ???-???, range 2-32 |
| --- | --- | --- |

| MEMADDR | Buffer RAM address | 0xA0000 - 0xF0000, range 4096-65536 |
| --- | --- | --- |

| TRANSCEIVER | Transceiver control | (internal) or 1 (external) |
| --- | --- | --- |