# Architectural Proposal

# LAN Manager Remote Access Services 2.x

Connectivity & Systems Integration Group

Authors:	Dialin Development Group

Revision 0.5 April 29 1992

Table of Contents

µ1.	Overview	1

2.	Key Architectural Paradigms	4

2.1.	Inter Networking Models	5

2.2.	Uniform RAS API (RASLIB)	5

2.3.	Modular Data Link Drivers	5

3.	RAS User Interfaces	6

3.1.	Administration User Interfaces	6

3.2.	Client User Interfaces	6

4.	RAS Server	6

4.1.	Supervisor	6

4.2.	Gateways	7

4.3.	Router Managers	7

4.4.	Authentication	7

5.	RAS Subsystem	7

5.1.	RAS Manager	8

5.1.1.	RAS API	9

5.2.	RAS Hub	9

5.3.	AsyMAC	10

5.4.	Binding	10

5.5.	Adding New Medias and Devices	12

5.5.1.    Adding different Medias	12

5.5.2.	Adding new Devices	12

5.6.	Supporting SNA	12

6.	RAS Clients	13

6.1.	Windows NT	13

6.2.	Windows 3.x	13

6.3.	MS-DOS 5	13

6.4.	OS/2	13

Open Issues

Get the client architecture

Describe LAN to LAN RAS (improved security)

Get agreement on the connection script architecture

Provide more detail on the UI architecture

·	Interfaces used

·	Remote Administration

·	WOW vs. Win32

·	Terminal mode

·	connection scripts

# 1	Overview

RAS provides a connectivity solution for remote PCs that want to access the services of a local area network.  RAS 2.x is essentially a port of RAS 1.0 functionality to the Windows NT platform, however, the system is re-architected to allow additional capabilities in future releases. Not all of the functionality described in this document will be implemented in RAS 2.0, rather version 2.0 will contain the minimum necessary functionality to ship a RAS product on NT as soon as possible.  Other point releases are expected to follow, so it is important to architect for the needs of those point releases now.  The RAS 2.x architecture is designed to support:

·	100% compatibility with down level RAS servers and down level RAS clients.

·	A framework which can integrate a set of network/transport protocols and a set of data link technologies. More specifically, RAS 2.x is architected to support:

·	any transport offering a NetBIOS interface;

·	a set of network protocols with routing capabilities: IP and IPX;

·	any  connection-based WAN data link: Async, X.25, ISDN, etc.

·	A modular and extensible structure. Support for different data links/transports will be modular, and the system will be configured only for the desired set.  Extensibility means that a new data link support module could be added in less than two months.

·	Data compression.

·	Authentication without requiring a specific network protocol.

·	Easy portability to future MS-DOS and Windows platforms.

·	Connectivity to Macintosh remote clients.

Although not strictly necessary, the reader is encouraged to become familiar with RAS 1.0 functionality, terminology, and model of operation.  For understanding the kernel level components of RAS 2.x, it is also recommended that the reader be familiar with the NDIS 3.0 interface.

·	For a detailed specification of the NDIS 3.0 interface, refer to the Microsoft LAN Manager Network Driver Interface Specification (NDIS) Version 3.0.

·	For an understanding of RAS 1.0 client architecture, refer to Microsoft LAN Manager Remote Access Service Remote Workstation Functional Specification Revision 1.24 by Patrick Awuah

·	For an understanding of RAS 1.0 server architecture, refer to Microsoft LAN Manager Remote Access Service Server Functional Specification Revision 1.25 by Patrick Awuah

RAS 2.x goals meet the growing needs of heterogeneous based networking.  Currently in RAS 1.0, only serial devices are supported as a network medium.  Furthermore, only AsyBEUI can run over the serial link.  As illustrated in Figure 1, RAS 2.x has far more potential.  It is architected to handle many media types including modems, X.25, ISDN, SNA LU 6.2, and even ethernet or arcnet.  It is also designed to handle multiple protocols, including AsyBEUI, IPX, TCP/IP, and others.

µ §

The RAS Server Service on NT can be viewed as three components, the User Interface, the RAS Server, and the RAS Subsystem.  The RAS User Interface is responsible for all user interaction with RAS including both a Windows based GUI, a command line for all platforms, and a character windows interface for MS-DOS and OS/2.  The RAS Server is designed to provide NetBIOS gateway, router management, and authentication services.  The RAS Subsystem provides connection services, and integrates third party drivers for a variety of media types.

µ §

# 2	Key Architectural Paradigms

The RAS 2.x architecture follows a few key architectural paradigms, the understanding of which is essential to the understanding of the benefits that RAS 2.x provides.  These include two inter networking models, or the model by which connections between remote clients and services on a LAN are made.  Also key is the concept of a uniform API available on all RAS platforms for allowing user mode applications to make use of RAS services.  Finally, RAS 2.x provides for the addition new types of data link media through an architected interface for data link drivers.

## 3	Inter Networking Models

RAS 2.x supports two inter networking models.  The first, the only model supported by RAS 1.0, is the NetBIOS gateway where RAS takes the NetBIOS requests of clients on the RAS network and submits them on the LAN.  In this model all network requests must be made through a NetBIOS interface, and RAS makes all requests on behalf of it clients.  The other inter networking model is the Network Router.  In this model RAS makes use of the routing capabilities of routable transports (TCP/IP, IPX, AppleTalk) by actually directing packets at the transport level.  Unlike the gateway model where RAS must interpret each NetBIOS command, the routing model allows the RAS network to pass packets to the LAN directly.

To illustrate the difference between these two models, Figure 3 shows the data flow in two typical cases.  The first case is an example of a client using RAS Gateway services.  The client uses AsyBEUI and a dial up phone line to connect to a NetBEUI resource on the Ethernet network.  The second case, a Network Router example, shows the same client using TCP/IP to connect to an ethernet resource.

µ §

## 4	Uniform RAS API (RASLIB)

Both the User Interface and the RAS Server need to control the operation of the RAS data links.  Moreover, the UI has to be implemented on a multitude of platforms each with its own operating environment.  In addition, applications may want to make use of a remote connection to a LAN using RAS.  For these reasons, a common interface called the RAS API for connection management between RAS networks and LAN networks is provided for all RAS platforms.  In version 2.0 this interface will not necessarily be exposed for third party applications.  However, it is architected so that it can be exposed in the future.

## 5	Modular Data Link Drivers

The RAS 2.x architecture allows for the inclusion of additional data link drivers given that the drivers implement either an NDIS interface or a standard serial interface.  A driver is provided as part of the RAS Subsystem to convert between the NT standard serial interface and an NDIS interface.

# 6	RAS User Interfaces

The user interface pieces are responsible for collecting and displaying RAS related information to the user.  Users are lumped into two groups, clients and admins, where clients are those who call the RAS Server from remote locations, and admins are those responsible for maintaining and monitoring the RAS server itself.

## 7	 Client User Interfaces

In NT RAS, much of the connection management logic has been moved from the client UIs into the RAS Manager.  This move isolates the client UIs from most platform dependent device and media behavior.  This is a critical modularization for two reasons.  First, the number of client platforms on which the visual and command line clients are required has expanded threefold.  Second, a third UI, the external APIs is now being provided.

### 8	Visual Client User Interfaces

The visual client interfaces (RASPHONEs) are Windows or character window programs (depending on platform) which give the user access to the connection, disconnection, and status functionality provided by the upper RAS Manager interface.  This functionality will be accessed for the most part through the external RAS APIs maximizing code reuse.

The visual client UI creates and maintains a "phone book" containing user-defined entries, where each entry includes connection parameters (phone numbers, PAD types, switch types, etc) required to establish a particular connection.

In addition to the basic RAS 1.0 functionality, the NT RAS visual client UI provides optional modem feature support, enhanced X.25 facilities support, and pre-connect and post-connect switch support.  The UI implements a connection over 1 to 4 intermediate connections (pre-connect switch to modem to X.25 PAD to post-connect switch).  A special "terminal" switch is provided allowing the user to interact directly with a pre- or post-connect device, such as a security box.

The NT RAS visual client UIs will automatically upgrade existing user phone book entries to a new expanded format.  Once this occurs, the RAS 1.0 programs will not be able to use the phone book.

There are 3 separate visual client programs, a COW-based program for DOS/Slalom, a Win16-based program for Win 3.x and Sparta, and a Win32-based program for Win NT.  The programs will be similar in appearance, operation, and functionality and will share a fair amount of code.  The 3 programs will all use the same phone book format.

### 9	 Command Line User Interfaces

The command line client interfaces (RASDIALs) are text-oriented programs which give the user the ability to establish and terminate RAS connections from the command line and from batch files.  Like the visual client UIs, the command line UIs will access the RAS Manager services for the most part through the external RAS APIs.

Unlike the visual client UIs, the command line UIs are not intended to be fully configurable.  For example, while limited "override" and "pre-prompt" capability is provided, the command line interface is generally dependent on the information contained in the phone book created with the visual client UI.  By it's very nature, the command-line UI cannot support interactive "terminal" switches when used in batch mode.  However, terminal support is provided in command line mode.

The command line interface will not be combined with the visual client interface for several reasons.  First, there is no reasonable way to merge the two in the Win 3.x environment which is a critical client platform, and merging for Win NT but not Win 3.x would be confusing for the user  Second, it is beneficial to provide a "low-memory" connection utility in the DOS environment, and the command-line UI fits this bill.

There are 2 separate command line client programs, a DOS-based program for DOS/Slalom, Win 3.x, and Sparta, and a Win32-based program for Win NT.  The programs will be similar in appearance, operation, and functionality and will share a fair amount of code.

### 10	 API Client User Interfaces

The API client interface is a small high-level set of dial, hangup, and status APIs exposed to the application developer as DLLs and import libraries.  The API client UI will allow the developer to establish and terminate RAS connections under program control.  Like the command-line UIs, the functionality provided is dependent on phone book entry information that must be created with the visual client UI.

There are 2 separate API client libraries, a Win16-based library for Win 3.x and Sparta, and a Win32-based library for Win NT.  A DOS-based library is possible, but is not included in the initial release.

## 11	Admin User Interfaces

The design and capabilities of the admin user interface do not change dramatically in NT RAS.

### 12	Visual Admin User Interface

The primary visual admin user interfaces (RASADMINs) are Windows programs that allow the user to monitor RAS users on a domain and RAS ports on a particular server.  The user can disconnect and send messages to RAS clients.  The user can start, stop, pause, and continue the remoteaccess service on the server.

The user can grant/revoke RAS permissions including setting preset callback numbers on RAS servers.  While RASADMIN will still provide this permission capability, it will also be provided in the User Manager (core NT) tool.  The User Manager will call a RASUSER DLL (provided by the RAS group) to set RAS permissions.

NT RASADMIN allows RAS servers to be administered remotely and is capable of administering old LM21 RAS servers.  LM21 RASADMIN will not be able to administer NT RAS servers.

For this release there will be only one visual admin user interface program, a Win32-based program for use on Win NT.  This means the RAS administrator must have an NT workstation.  This is not ideal, especially since a Win 3.x program was provided in RAS 1.0.  However, the core NT admin UIs (Computer Manager, User Manager, etc) also have this restriction, so NT admins at least will already need an NT workstation.

In the future a Win 3.x version of NT RASADMIN and the RASUSER DLL will be provided.  This will likely occur in the same time frame that the core NT admin UIs migrate back.

### 13	Command Line Admin Interface

Command line access to admin functionality is not included in this release and there is no significant work planned that would make this easier in the future.

### 14	API Admin User Interface

The API admin user interface will not be exposed in this release.  However, a set of RasAdmin APIs encapsulating the majority of the admin functionality will be used internally.

## 15	 Setup User Interface

Setup will be provided in at least two programs, one for NT and at least one for DOS, Win 3.x, Sparta, and Slalom.  Only NT Setup will setup the RAS Server.  Both setups will set up RAS clients.

NT Setup will be a GUI script based on (and integrated with) the installation/configuration utility for NT LAN Manager being implemented by David Hovel of the NetUI group.  NT Setup will register RAS in the NT Registry for configuration management purposes and will add the information needed to start the remoteaccess service in NT.

The client setup will appear basically the same as the RAS 1.0 setup, except that local X.25 PADs will be supported as devices in addition to modems and additional questions about compression will be necessary.

# 16	RAS Server

The RAS Server is the component that provides NetBIOS gateway services, manages routing tables, connects remote clients with the appropriate RAS services (which gateway to use, which routing manager to use, etc.), provides authentication of remote clients, and manages RAS connections.  It will be written to the Win32 API to provide for portability to MS-DOS 6 and Windows 3.2 and above platforms.  The server modules can be grouped into a number of functional areas including the Supervisor, Authentication, Gateways, and Router Managers.  The Supervisor manages data link connections, provides an interface to the LAN Manager service controller, and invokes the requested RAS services for RAS clients.  The authentication module provides authentication services for RAS clients.  The Gateway modules implement the NetBIOS gateway functionality.  Finally, the Router Manager modules modify protocol routing tables as connections to RAS clients are created and destroyed.

## 17	Supervisor

The Supervisor is the central module of the RAS 2.x Server.  It provides the LAN Manager Service Controller communication channels, invokes the necessary RAS Services on behalf of RAS clients, and provides an interface for administration by the Admin UI.  The supervisor is invoked by the LAN Manager Service Controller when a request to start the RAS service is made.  The Supervisor gathers configuration information from the NT Registry, starts the appropriate processes for Gateway services and invokes the appropriate DLLs for Router services.  It then enumerates the set of RAS endpoints and listens for connections on those endpoints.  Once a connection is requested, the Supervisor gets the first frame sent by the remote client. Based on the contents of that frame, the Supervisor determines which authentication scheme to use and which RAS Service to invoke (one of the NetBIOS gateways or one of the router managers)  Once the connection is broken, the Supervisor informs the appropriate module to clean up the connection.

## 18	Gateways

There are plans for two types of NetBIOS gateways.  First, the RAS 1.0 gateway functionality will be implemented as a port of the RAS 1.0 gateway.  This gateway has the disadvantage that it doesn't preserve end to end semantics for the NetBIOS NETSEND command.  The other gateway planned is an RPC NetBIOS gateway that will preserve the end to end semantics of NetBIOS.  It is not likely that the RPC NetBIOS gateway will be implemented for version 2.0

## 19	Router Managers

RAS 2.0 will not support routable protocols.  The concept of a Router Manager is only mentioned in this document to show that the server component can be extended in such a way as to manage clients who want to use a routable protocol instead of a gateway.  Support for routable protocols may be added as a future enhancement.

## 20	Authentication

RAS 1.0 only supported AsyBEUI clients, hence the authentication exchange in RAS 1.0 was carried out over AsyBEUI.  In RAS 2.x however, there is no guarantee that the client will have the AsyBEUI protocol (it could be TCP/IP or IPX for example).  For this reason, authentication has been enhanced in RAS 2.x to not require a particular protocol on the client.

Once the RAS Server has initialized, it posts listens on each of the active RAS endpoints using the RAS API.  When a client wants to connect to the server it calls the RAS API RasConnect() which causes the listen to be signaled.  The RAS Server then reads the first frame and determines if the frame takes the form of a 1.0 client or a 2.x client.  Frames for 1.0 clients are AsyBEUI frames, so when the Server determines that the client is a 1.0 client it posts a NetBIOS listen on the AsyBEUI protocol stack and uses the RAS API to connect the client endpoint to the AsyBEUI stack.  As the client re transmits the first frame a few times, the listen on the servers AsyBEUI stack gets signaled, and then the RAS Server carries out an authentication conversation in exactly the same manner as RAS 1.0.

If instead the RAS Server determines from the first frame that this is a RAS 2.x client, then the RAS Server uses the RAS API to read and write data to the client directly.  These API do not provide for guaranteed delivery.  An authentication conversation is carried out, and if authenticated, the RAS server uses the RAS API to connect the server end of the RAS endpoint to the appropriate protocol.

# 21	RAS Subsystem

The RAS Subsystem presents a robust way of dealing with heterogeneous RAS and LAN media types.  It does so by providing connection and routing management at the user level (RASLIB) and the layers below it (RAS Manager, RAS Hub, AsyMAC, and NDIS).

The main goal of RAS is to provide remote clients the ability to connect to a LAN using whatever connectivity happens to be available between the client and the LAN.  Ideally, RAS should be able to support a wide variety of media and transport types. This includes both connection (X.25 and modems) and connection less (ethernet) oriented, reliable (X.25, V.42) and unreliable (modems) media types.  The RAS subsystem is architected to handle this potpourri of possibilities.  To summarize, the goals of the RAS Subsystem are to:

·	Provide a standard programmatic user mode interface to manage connections between RAS endpoints and LAN protocols

·	Provide an extensible framework so that new media types can be easily added to the existing system.

·	Provide configuration capabilities for the various RAS media.

·	Provide easy portability to MS-DOS 6.0 and Sparta.

·	Provide Macintosh client connectivity utilizing Macintosh client software.

·	Compression

The following figure illustrates the various components of the RAS Subsystem and the interfaces they use and export.  Each component is described below.

µ §

## 22	RAS Manager

The RAS Manager is a user mode process that implements the RAS API, a set of API available to user mode applications and used by both the RAS Server and the RAS UI to make use of RAS services.  The RAS manager abstracts the various types of RAS endpoints so that processes that make use of RAS Manager don't have to be aware of the media type specific nature of each endpoint.  This is done through the use of Media DLLs.  For each media type, a DLL is implemented that is used to control endpoints of that media type. In this manner, a process can call RAS Manager to make a connection over a particular endpoint. For example, if the endpoint happens to be an X.25 endpoint, then the X25 DLL is invoked to do the X.25 specific actions necessary to make a connection. Similarly, a Serial DLL would be invoked for communication through a COM port.  All Media DLLs export a set of APIs; the semantics of these APIs allow enough abstraction so that RAS Manager itself can be independent of Media specifics (for example these APIs may be PortOpen(), PortEnum(), PortConnect(), etc.). In addition to Media DLLs, RAS Manager may use Device DLLs to talk to intermediate Devices such as Modems, switches, X25 PADs etc. While the Media DLLs encapsulate the knowledge of how to control/communicate over different Media (for example, using X25Send() in X25 environment), the Device DLLs contain knowledge of how to talk to the different intermediate devices. This knowledge includes things like script files, read/write time characteristics, etc. Like the Media DLLs, all Device DLLs must export a set of APIs (for example, DeviceConnect(), DeviceListen(), etc.). In the course of establishing a connection the user may connect through zero, one or more devices.

The RAS Manager exports an interface called the RAS API to user mode applications. RAS Manager also makes use of the Hub interface discussed in the RAS Hub section below.  This interface is used to among other things, associate a particular endpoint with a particular network on a protocol stack.

### 23	RAS Manager APIs

While by no means complete or finalized, here is a flavor of the type of API exported by the RAS Manager:

·	RasPortEnum(OUT pBuffer, IN cbBuffer) ;	Places in the buffer pointed to by pBuffer a data structure that describes all available configuration information about all available RAS endpoints.  cbBuffer is the size in bytes of the buffer provided by the caller

·	RasPortGetInfo(IN hport, OUT pBuffer, IN cbBuffer) ;	Provides information about a particular RAS endpoint referred to by the endpoint handle, hport.  The information is placed in a buffer pointed to by pBuffer whose size is measured in bytes and passed as cbBuffer.

·	RasDeviceConnect(IN hport, IN pInfo, IN timeout, IN hSem) ;	Requests that a connection be made over the RAS endpoint referred to by the endpoint handle, hport. Information about what to connect to is passed in pInfo, and the timeout for the connection request is passed in timeout.  RasDeviceConnect returns immediately, and when the connection has been established, the semaphore referenced by the semaphore handle hSem is signaled.

·	RasPortListen(IN hport, IN pInfo, IN timeout, IN hSem) ;	Allows the application to wait for a connection request on the endpoint whose handle is hport.  An asynchronous call, RasPortListen will signal the semaphore whose handle is hSem when the connection is requested. Note that this API is called RasPortListen() as contrasted with RasDeviceConnect(), this is because there can be a "listen" posted only with the endpoint (port), while the user may wish to do through several devices to connect.

·	RasPortDisconnect(IN hport) ;	Allows an application to disconnect a RAS session happening on the endpoint whose handle is hRpd.

·	RasPortOpen(IN hport, IN DesiredAccess) ;	Allows an application to open a particular RAS endpoint previously enumerated with RasPortEnum().  DesiredAccess is an access mask for the desired access to the endpoint which currently can be RAS_DIALIN or RAS_DIALOUT.

·	RasPortClose(IN hport) ;	Allows an application to close an open RAS endpoint and free the resources allocated to it.  hport is the handle of the endpoint that will be closed.

·	RasPortGetStatistics(IN hport, OUT pInfo, IN cbInfo) ;	Allows an application to gather statistics about a particular endpoint.  hport is the handle to the endpoint that statistics are desired for.  pInfo is a pointer to a buffer that the statistics will be placed in, and cbInfo is the size in bytes of the buffer.

·	RasPortSend(IN hport, IN pData, IN cbData, OUT pcbSent, IN timeout) ;	Allows an application to send data synchronously over a RAS connection.  hport is the handle to the local endpoint of the connection.  pData points to the data to be sent.  cbData is the size of the data buffer in bytes to be sent.  pcbSent is a pointer to an integer that will contain the number of bytes transferred when the call completes.  timeout is the number of seconds to allow for the data transfer.

·	RasPortReceive(IN hport, IN pBuffer, IN cbBuffer, OUT pcbRead, IN timeout) ;	Allows an application to receive data synchronously over a RAS connection.  hport is the handle to the local endpoint of the connection.  pBuffer points to a buffer to receive the data, and cbBuffer is the size, in bytes, of the buffer.  pcbRead is a pointer to an integer that will contain the count of bytes received when the call completes, and timeout allows the user to specify a maximum number of seconds to wait for the data.

## 24	RAS Hub

The key function of the RAS Subsystem is to route frames from a heterogeneous collection of RAS endpoints to a heterogeneous collection of transports.  The component that implements this function is the RAS Hub.  The RAS Hub is implemented as a device driver that exposes an NDIS interface at the top and uses an NDIS interface at the bottom.  To protocols, the RAS Hub behaves like a MAC.  To MACs, the RAS Hub behaves like a protocol.  A MAC that binds to RAS Hub is called a RAS MAC, and remote clients enter the network through these MACs.

In addition to the NDIS interface that RAS Hub exports and uses, RAS Hub exports a control interface called the Hub interface to allow the RAS Manager to indicate which RAS endpoints should be routed to which protocols.  This interface is used to set up each connection.  When a connection is made, the Server determines the protocol that the remote client is requesting and to route the endpoint to a particular protocol..  Moreover, the Hub interface provides a method by which the RAS Manager can query about configuration information including the number of RAS endpoints configured and the names of the media specific DLLs used.  This is how RAS Manager determines at startup time the set of endpoints it supports, and the DLLs it has to use to interact with those endpoints.

## 25	AsyMAC

The RAS Subsystem allows for the integration of RAS media drivers that take two forms.  A driver that controls a type of RAS endpoint can either export the NT serial I/O interface, or it can export the NDIS 3.0 interface.  If the driver exports a serial interface, then the AsyMAC driver is used to convert between the serial interface and an NDIS interface.  In this case, serial drivers exchange data with AsyMAC over a serial interface, and AsyMAC binds to RAS Hub using NDIS.  The binding between AsyMAC and RAS Hub is unique in that it binds to RAS Hub only once even though it may be supporting multiple ports over multiple drivers.  AsyMAC implements logic to multiplex between the one RAS Hub binding and the multiple serial interface bindings below it.  Moreover, as drivers for slow speed links are typically implemented as serial drivers, compression and decompression will be implemented in the AsyMAC driver.

## 26	Binding

As discussed earlier, the RAS Hub is unique in that while it only has one binding to each of the 'MACs' below it, those MACs can support multiple endpoints.  For example, the single AsyMAC driver can support a combination of X.25 and serial endpoints.  RAS Hub keeps information about the individual endpoints supported by each MAC below it, and this information is referenced by a common endpoint handle known by RAS Manager, RAS Hub, and AsyMAC.  When a connection is made the handle is included as part of the unique IEEE address for each endpoint.  When RAS Manager sends data to a particular endpoint, it uses a particular endpoint handle that RAS Hub knows about.  From the handle, RAS hub can determine which MAC to route the data to, and the data is routed with the handle as part of the IEEE address.  The MAC driver can then reference the address to determine which specific port to send the data on.

The mapping of RAS handles to endpoints is generated when the RAS subsystem loads.  The details on how these handles are maintained and how port specific data is gathered from the registry will be discussed in detail in the component design document for the RAS Subsystem.

## 27	Adding New Medias and Devices

This section tells how new medias like ISDN, Frame Relay, etc. can be added to work with RAS. It also discusses how new devices can be added without any change to the RAS code.

## 5.5.1.	Supporting different Medias

The only media dependent components in RAS are the UI, Media DLLs, and the MAC. The UI contains knowledge of the different parameters that the Media DLL may need for establishing a connection (for example, Baud Rate in case of Serial Media, etc., X25 Address in case of a X25 Media). The Media DLLs map the APIs they export to the Media specific APIs (for example, the PortSetInfo() API is translated to SetDCBInfo() API in case of Serial Media). Lastly, the MAC knows how to talk to the port (endpoint) once the connection is established and the "frame talk" is underway.

New Medias can be added in several ways: if the Media can "appear" like an existing Media then no new development work is required to support it. For example, the X25 card manufacturers have written drivers that conform to the COM driver spec. In this case the X25 Media appears like Serial Media to RAS, and the existing Serial DLL is used unaltered. (There may be some UI work required to support Media specific parameters; in this example, it is not required since the RAS has support for X25 PADs in the UI).

Another way for adding new medias is to write new Media DLL and a MAC driver. This is attractive when already an NDIS 3.0 compliant MAC exists; only a Media DLL may be written to support connection establishment over the Media. Like above, there may be some UI changes required.

## 5.5.2.	Supporting new devices

RAS Manager loads Device DLLs by name, for example, if connection is requested through a device called FOO, it will try to load FOO.DLL. This extensibility makes it possible to add new devices dynamically. Different devices may require some UI changes, or may be able to use the existing UI with alternative script files for connection establishment.

## 28	Supporting SNA

This is a detailed example of how RAS can be made to work with SNA Media.

RAS connections over SNA networks will be supported through the addition of a RasSNA DLL (media specific DLL described earlier), a RasSNA MAC, and the APPC services provided by the Comm Server group.  The RasSNA DLL will accept connection requests from RAS Manager, and will translate those requests into APPC requests.  When a connection is established over the SNA network, the RasSNA DLL will signal the RasSNA MAC so that it can start to send and receive packets. Since the APPC services only have a user mode interface, the RasSNA MAC writes frames to a file which is then read by a user mode thread in the RasSNA DLL and sent using the APPC Send() APIs. Similarly, a user mode thread in the DLL reads frames coming across through the APPC Read() API and writes it to a file which is then read by the MAC.

This architecture is described in the figure below:

µ §

# 29	RAS Clients

## 30	Windows NT

## 31	Windows 3.x

## 32	MS-DOS 5

## 33	OS/2

Modification History

| Date | Done By | Reviewed By | Comment |
| --- | --- | --- | --- |
| 2/21/92 | FrankB |  | Adapted from original documents by Stefan and Thomas. |
| 3/12/92 | FrankB | JawadK | Modified to include discussion on authentication and data link specific DLLs. |
| 3/13/92 | FrankB |  | Incorporated Jawad's review comments |
| 4/23/92 | SteveC |  | Filled out User Interface discussion |
| 4/29/92 | FrankB |  | Added RasSNA discussion |
| 6/10/92 | GurdeeP |  | Added information on Media and Device DLLs and how to add new Medias. Also edited RasSNA discussion. |

X.25 is supported through third party devices that provide a serial interface to X.25 PADs.

On NT, the GUI connection application will accept command line arguments; a separate RASDIAL command line will not be implemented.

Routable transports will not be supported in the first release of RAS 2.0