WINDOWS NT 5.0 UNIDRV CONTROL MODULE

DESIGN SPECIFICATION

Version 0.7

7/29/26

Table of Contents

1. INTRODUCTION

1.1  ASSUMPTION

2. ARCHITECTURE OVERVIEW

3. INTERFACES

3.1 INTERFACE WITH GDI

3.1.1 DDI EXPORTS

3.2 INTERFACE WITH VECTOR MODULE

3.2.1  VectorInit( )

3.2.2.  VectorDisableSurface ( )

3.2.3.  VectorDisablePDEV ( )

3.2.4. Vector Functions

3.3 INTERFACE WITH RASTER MODULE

3.3.1  RasterInit ( )

3.3.2.  RasterDisableSurface ( )

3.3.3.  RasterDisablePDEV ( )

3.3.4  Raster Functions

3.4 INTERFACE WITH FONT MODULE

3.4.1  FontInit( )

3.4.2.  FontDisableSurface ( )

3.4.3.  FontDisablePDEV ( )

3.4.4 Font Functions

3.5  DrvMemAlloc - DrvMemFree

3.6 INTERFACE WITH OEM CUSTOM UI

4.  PRINTING MODE

4.1  Vector vs. Raster Mode

4.2  Banding vs. Non-Banding Mode

5.  INTERNAL BINARY DATA

5.1   INFOHEADER

5.2  DRIVERINFO

5.3   COMMAND

5.4  PARAMETER

5.5  ValueStack/Operator Stack Array

5.6 SEQUENCE COMMAND

5.7 ORDERDEPENDENCY

5.8  STATE VARIABLES

5.9  GLOBAL ATTRIBUTES AND COMMANDS

5.9.1 GLOBAL ATTRIBUTES

9.5.2 GLOBAL COMMANDS

5.10 RENDERING INFORMATION FOR PREDEFINED FEATURES

6.   DEVINFO

7.  GDIINFO

8. PDEVICE

8.1  UNIDRV PDEVICE

8.  DEVMODE

8.1 PUBLIC DEVMODE

8.2 PRIVATE DEVMODE

9. PRE-ANALYSIS

9.1  GDI Requirements

9.2  GDI performs Pre-Analysis

9.3  Driver performs Pre-Analysis

9.3.1  Determination for enabling pre-analysis

9.3.2 Report all capabilities to GDI

9.3.3 Determination of color or monochrome object

9.3.4  Building of linked list of objects’ bounding boxes.

9.3.5  Z-Ordering Detection

9.3.6 Real rendering pass

10.  OPEN ISSUES

# 1.	INTRODUCTION

This document describes the design specification and implementation details of the Control module for Unidrv5.  It includes description of interfaces and data structure requirements.  This is a working document and will be updated as the project progresses.

The Control module is the printer graphics driver DLL seen by GDI.  Its purpose is to provide device specific information to GDI, dispatches GDI  drawing requests to the appropriate font/raster/vector module, manage the PDEVICE,  marks the ending and starting point of a page/job, handles page error recovery, performs banding, performs pre-analysis, realizes device compatible objects such as font, brushes, palettes and sends printer initialization instructions to the printer.

## 1.1  ASSUMPTION

This document assumes that GDI will support the following in NT 5.0

- Ability for the driver to switch surface on a per band basis.

- Ability to change what functions are hooked on a per band basis.

- Ability to change device capabilities on a per band basis.  This is necessary to solve the z-ordering problem with device fonts.  If the driver detects z-ordering problem during pre-analysis pass, it must have the capability to report that it doesn’t support device font so the driver would not get device fonts during the real rendering pass.  At that point the driver can perform print text as graphics.

# 2.	ARCHITECTURE OVERVIEW

The following figure illustrates the interface of the Control module.

CONTROL MODULE

This module handles requests to enable/disable the driver, returns device capabilities, returns driver capabilities, initializes/manages PDEVICE, realizes logical objects to device compatible objects and directs drawing requests from applications.  It also controls the flow of the job, handles error recovery, performs pre-analysis of data,  and calls the vector/raster/font/OEM modules to perform drawing operations.

VECTOR MODULE

This module is called by the Control module to handle all vector graphics operations.  This module is called vector printing is selected.  It converts all DDI graphic drawing calls to equivalent vector commands to be sent to the printer device.

RASTER MODULE

This module is called by the Control module to handle all the raster graphics operations.  This module is called for printer devices that do not have vector support or when raster printing is selected.  It rasterizes the DDI drawing commands into the banding buffer to send to the printer device.

FONT MODULE

This module is called by the Control module to handles all font/text related operations and requests.  These include enumerating device fonts, initializing default fonts, downloading TT fonts to printer, handling text drawing operations such as DrvTextOut.

OEM CUSTOM DLL

This module is called by the Control module to allow OEM to hook in specific DDI or Unidrv callback function for special handling.  The OEM would be able to influence Unidrv’s process in certain ways and to inject special byte sequence into the output data stream.  (This module is optional)

INTERNAL BINARY DATA/PRINTERDATA/DEVMODE

Internal Binary Data encapsulates all the printer information necessary to display the graphical user interface, to render and send the print job to the printer.  The GPD parser parses the text based GPD file and builds the internal binary data structure.  This structure is cached in a binary file and shared between the UI module and the Control module.

PrinterData contains all the printer-sticky information for the device.  There is a PRINTERDATA struct associated with every installed printer device.  This structure is stored in the registry and shared between UI module and the Control module.  Any changes to printer-sticky features in the UI are reflected in the this structure.

Devmode contains all document sticky information for the job.

# 3.	INTERFACES

The Control module provides the DDI interface to GDI.  However, it does not personally process all requests, it calls the Font Module to handle all text/font related requests, the Raster Module/Vector Module for graphics requests (depending on the selected print mode), and the OEM custom DLL for any customization.

## 3.1	INTERFACE WITH GDI

In the Windows NT GDI model, a printer driver controls the output by implementing as much DDI as necessary for the device, the rest of the DDI are supported by GDI.  There are two types of DRV export functions, required driver functions and optional driver functions.  Optional driver functions are available for printer drivers that want to manage the device surface as well as optimize the graphics output for the devices.  The driver is required to export only two functions, DrvEnableDriver and DrvDisableDriver.  All other driver supported functions are exposed through a DRVFN table.  DRVFN table is an array of index-function pointers.  GDI calls DrvEnableDriver to get the number of driver supported DDI and get the pointer to the DRVFN table.

It is possible for the driver to export all printing related DDI functions and delays the decision of what function to hook at EngAssociateSurface time.  This GDI function allows the driver to specify which DDI function it wants to perform on the surface created.  The Control module also communicates with GDI via. service routines.  The Control module can always call GDI’s EngXXX function for requests that it cannot handle or fails to perform.

The following figure illustrates the interface between the Control Module and GDI.

### 3.1.1	DDI EXPORTS

In addition to DrvEnableDriver and DrvDisableDriver, the Control module will expose all printing related DDIs through the DRVFN table.  However, it will not be required to handle all exposed functions. It has the ability to decide which function needs to be hooked out at DrvEnableSurface time.  However, the hook flags are limited to the following functions (DrvBitBlt, DrvStretchBlt, DrvTextOut, DrvPaint, DrvStrokePath, DrvStrokeAndFillPath, DrvLineTo, DrvCopyBits, DrvSynchronize).

Unidrv will support vector mode as well as raster printing mode.  The printing mode is dependent on the user’s selection (or the pre-analysis result)and the device capabilities.  The table below outlines the function needed to hooked in either raster or vector printing mode.

Flag							Function to be hooked

HOOK_BITBLT						DrvBitBlt (Raster, Vector)

HOOK_STRETCHBLT					DrvStretchBlt (Raster, Vector)

HOOK_TEXTOUT					DrvTextOut (Raster, Vector)

HOOK_PAINT						DrvPaint (Vector)

HOOK_STROKEPATH					DrvStrokePath (Vector only)

HOOK_FILLPATH					DrvFillPath (Vector only)

HOOK_STROKEANDFILLPATH				DrvStrokeAndFillPath (Vector only)

HOOK_LINETO						DrvLineTo (Optional)

HOOK_COPYBITS					DrvCopyBits (Raster, Vector)

The following is a list of exports for the Control module as defined in the DRVFN table.  It includes descriptions of the tasks performed by the Control module.   Where appropriate, the Control module will call the Vector Module, Font Module, Raster Module or OEM Custom Module.  Refer to the Win32 SDK for prototypes of these DDIs.

DrvEnableDriver

This function fills in DRIVERENABLEDATA structure with the driver version, number of export functions and the pointer to the DRVFN table.  It also call the Font/Vector/Raster/OEM DLL  modules for initialization.

DrvDisableDriver

This function is called before the engine unloads the driver to allow cleaning up of allocated resources.

DrvEnablePDEV

This function fills in the GDIINFO struct with device capabilities, fills the DEVINFO struct with driver capabilities and allocates/initializes the PDEVICE.

DrvResetPDEV

This function transfers the state from the old PDEVICE to the new PDEVICE when the applications call ResetDC().  It also checks for changes in paper size , paper source and orientation of the new PDEVICE so it can send new page initialization commands (if necessary).

DrvCompletePDEV

This function is called when the installation of the physical device is completed.  The Control module keep the GDI PDEVICE (HDEV) handle passed in for future calls to GDI services.

DrvDisablePDEV

This function frees up memory allocated for the PDEVICE.  In addition, the Control module will call FontDisablePDEV and either RasterDisablePDEV or VectorDisablePDEV to allow these modules to perform any internal clean-ups and memory deallocation associated with the PDEVICE.

DrvEnableSurface

This function calculates the amount of memory required for the surface and call EngCreateDeviceSurface for non-banding (vector) mode and EngCreateBitmap for banding (raster) mode.  Once that is completed, it calls EngAssociateSurface to associate the surface with the PDEVICE.  The driver also set the hook flags to specify the DDI function it want to hook as appropriate for the print mode selected.  In the case of banding, the driver calls EngMarkBandingSurface before calling EngAssociateSurface.

DrvDisableSurface

This function frees all memory allocated for the surface.  It also calls EngDeleteSurface to delete any surface associated with the device.  In addition, the Control module will call FontDisableSurface and either RasterDisableSurface or VectorDisableSurface to allow these modules to perform any internal clean-ups and memory deallocation associated with the surface.

DrvStartDoc

This function sends the Job_Setup sequence commands, as specified in DRIVERINFO.  See internal binary data section for more details.  Job_Setup commands are commands that need to be sent at the start of every job.

DrvEndDoc

This function sends the control commands to specify the end of a job (Doc_Finish  and Job_Finish sequence commands).  In the case of an aborted job, AbortDoc command will be sent.  If there is no AbortDoc command specified, the driver will just close the spool file and clean up internally.

DrvStartPage

This function performs a number of tasks to start the page.  It sends the control commands to start a document (only for first page of the document, Doc_Setup sequence commands) and a page(Page_Setup sequence commands). In the case where the device is a serial device with font support, it will call the Font Module to initialize the header of the sorting list.  Note:  the sorting list is necessary for serial printers with font support because glyph needs to be send down according to their position on the page since serial printers print unidirectional.

DrvStartBand

This function prepares for banding and returns the origin of the first band.

DrvNextBand

This function renders the content of the band just drawn and returns the origin of the next band. This function returns

(-1, -1) to indicate the end of the page.

DrvSendPage

This function is called when GDI completed the drawing for the page.  The control module calls the Raster Module dump function to dump the rendered data to the printer.  It does not need to dump anything if the print mode is vector since vector commands are sent to the printer as they are created.  This function also sends the end page commands to end the page(Page_Finish sequence commands).

DrvEscape

This function supports three escapes QUERYESCSUPPORT, PASSTHROUGH and SETCOPYCOUNT. PASSTHROUGH allows applications to bypass the driver and send the raw data directly to the printer.  SETCOPYCOUNT sets the driver copy count by overwriting the DEVMODE.dmCopies value.

DrvDitherColor

This function is called to request the driver to dither brush based on the device palette.  The Control module returns DCR_HALFTONE, requesting the engine to create a halftone approximation for the brush for the driver or DCR_DRIVER if Unidrv or GPD provided the dithering pattern.

DrvQueryFont

This function is called to fills the IMIMETRICS structure for a given font.  The Control module calls the Font Module for handling.

DrvQueryFontTree

This function is called to return a tree structure describing the mapping between UNICODE and printer glyphs or a tree structure describing the mapping between kernings.  The Control module calls the Font Module for handling.

DrvQueryFontData

This function is called to retrieves information about glyphs or kerning pairs.  The Control module calls the Font Module for handling.

DrvGetGlyphMode

This function is called to query how GDI should cache glyph data for a particular font.  The Control module calls the Font Module for handling.

DrvFontManagement

This function is called by GDI whenever an application calls ExtEscape  for QUERYESCSUPPORT, GETTEXTENDEDMETRICS.  The Control module calls the Font Module for handling.

DrvQueryAdvancedWidth

This function is called to obtain character advanced widths for a specified set of glyph.  The Control module will call the Font Module for handling.

DrvTextOut

This function is called to perform all output of text.  The Control module calls the Font Module FMTextOut interface for all text handling.

DrvBitBlt

This function is called to perform bit-block transfer to the device surface.  The Control module calls the Raster Module or Vector Module for bitblt handling.   HOOK_BITBLT is always set at EngineAssociateSurface.

DrvStretchBlt

This function is called to perform stretching bit-block transfer to the device surface.  The Control module calls the Raster Module or the Vector Module(if appropriate) for handling.  HOOK_STRETCHBLT is always set at EngineAssociateSurface.

DrvCopyBits

This function is called to perform the translation between device managed raster surfaces and GDI standard format bitmaps. HOOK_COPYBITS is always set at EngineAssociateSurface. Should this function call only Raster Module for handling??

DrvStrokePath

This function is called to stroke a path.  The Control module will call the Vector module StrokePath function to handle the request.  HOOK_STROKEPATH is set only if the printing mode is Vector mode.  To support stroke path the Control module must set the following flags in DEVINFO.flGraphicsCaps: GCAPS_BEZIER | GCAP_GEOMETRICWIDE.

Some printers such as HP GL/2 do not support generic path objects.  They support specific shapes such as rectangles, circles ect.  For each path object call, the Vector module must determine whether it can handle it.  If it cannot handle it, it should return FALSE so GDI will simulate it into simpler calls.

DrvFillPath

This function is called to fill a path on a device-managed surface.  HOOK_FILLPATH is set only if the printing mode is Vector mode.  To support fill path (for device-managed surface) the Control module must set the following flags in DEVINFO.flGraphicsCaps: GCAPS_BEZIER | GCAPS_ALTERNATEFILL | GCAPS_WINGDINGFILL.  The Control module calls Vector module FillPath function to handle the request.

DrvStrokeAndFillPath

This function is called to stroke and fill a path.  HOOK_STROKEANDFILLPATH is only set if the printing mode is Vector mode.  To support stroke and fill path, the Control module must set the following flags in DEVINFO.flGraphicsCaps: GCAPS_BEZIER | GCAPS_ALTERNATEFILL | GCAPS_WINGDINGFIL | GCAPS_GEOMETRICWIDE.  The Control module calls Vector module StrokeAndFillPath to handle the request.

DrvRealizeBrush

This function is called to realize a logical brush into a physical brush.  The Control module will call either the RasterRealizeBrush or VectorRealizeBrush, depending on the printing mode.

DrvDrawEscape

This function supports device specific drawing requests.

## 3.2	INTERFACE WITH VECTOR MODULE

### 3.2.1  VectorInit( )

In general, the vector module will send path objects to printers that support paths.  For other printers that do not support paths, the vector module will have to simulate to paths to something like LineTo command.

BOOL  VectorInit(PDEVICE lpdv, DEVINFO * lpDevInfo, GDIINFO *lpGDIInfo )

This function is called from the Control module to intialize PDEVICE, DEVINFO, GDIINFO.  The Control module calls VectorInit at DrvEnablePDEV time if the print mode is Vector or pre-analysis.  The Control module uses the information initialized to return to GDI and for future processing.

ITEMS IN PDEVICE THAT NEED INITIALIZATION BY VECTOR MODULE

HOOK FLAGS

lpdv->flHook could be any combination of the following flags.

HOOK_FILLPATH

HOOK_LINETO

HOOK_STROKEPATH

HOOK_STROKEANDFILLPATH

HOOK_REALIZEBRUSH

HOOK_BITBLT

HOOK_STRETCHBLT

HOOK_COPYBITS

HOOK_PAINT

Returns: This function returns TRUE for success and FALSE for failure.

### 3.2.2.  VectorDisableSurface ( )

This function is called by the Control module at DrvDisableSurface to allow the Vector module to clean up its internal data and to deallocate any memory associated with the surface.

BOOL	VectorDisableSurface(PDEVICE lpdv);

Returns: TRUE for success and FALSE for failure.

### 3.2.3.  VectorDisablePDEV ( )

This function is called by the Control module at DrvDisablePDEV to allow the Vector module to clean up its internal data and deallocate any memory associated with the PDEVICE.

BOOL	VectorDisablePDEV(PDEVICE lpdv);

Returns: TRUE for success and FALSE for failure.

### 3.2.4. Vector Functions

The following are possible export functions for the Vector module.  The Vector module must, at a minimum, exports VectorFillPath, VectorStrokePath, VectorStrokeAndFillPath and VectorRealizeBrush.  The function prototypes for the exported Vector functions should match those equivalent DDIs defined in Win32 DDK.

VectorFillPath ( )

VectorLineTo ( )

VectorStrokePath ( )

VectorStrokeAndFillPath ( )

VectorRealizeBrush ( ) - Required export.

VectorBitBlt ( )

VectorStretchBlt ( )

VectorCopyBits ( )

VectorPaint ( )

## 3.3	INTERFACE WITH RASTER MODULE

### 3.3.1  RasterInit ( )

BOOL  RasterInit(PDEVICE lpdv, DEVINFO * lpDevInfo, GDIINFO *lpGDIInfo )

This function is called from the Control module to intialize PDEVICE, DEVINFO, GDIINFO.  The Control module calls RasterInit at DrvEnablePDEV time if the print mode is Raster.  The Control module uses the information initialized to return to GDI and for future processing.

ITEMS IN PDEVICE THAT NEED TO BE INITIALIZED BY RASTER MODULE

HOOK_FLAGS

lpdv->flHook could be any combination of the following flags.

HOOK_BITBLT

HOOK_STRETCHBLT

HOOK_COPYBITS

Returns:  This function returns TRUE for success and FALSE for failure.

### 3.3.2.  RasterDisableSurface ( )

This function is called by the Control module at DrvDisableSurface to allow the Raster module to clean up its internal data and to deallocate any memory associated with the surface.

BOOL	RasterDisableSurface(PDEVICE lpdv);

Returns: TRUE for success and FALSE for failure.

### 3.3.3.  RasterDisablePDEV ( )

This function is called by the Control module at DrvDisablePDEV to allow the Raster module to clean up its internal data and deallocate any memory associated with the PDEVICE.

BOOL	RasterDisablePDEV(PDEVICE lpdv);

Returns: TRUE for success and FALSE for failure.

### 3.3.4  Raster Functions

The following are possible export functions for the Raster module. The function prototypes for the exported Raster functions are the same as those equivalent DDIs defined in Win32 DDK.

RasterBitBlt ( )

RasterStretchBlt ( )

RasterCopyBits ( )

## 3.4	INTERFACE WITH FONT MODULE

The Font module handles all text related DDI.  It also enumerates device fonts and downloads TrueType fonts to the printers.  The Control module calls the Font module to perform the text related tasks.

### 3.4.1  FontInit( )

BOOL  FontInit(PDEVICE lpdv, DEVINFO * lpDevInfo, GDIINFO *lpGDIInfo )

This function is called from the Control module to intialize PDEVICE, DEVINFO, GDIINFO.  The Control module always call FontInit at DrvEnablePDEV time.  The Control module uses the information initialized to return to GDI and for future processing.

ITEMS IN PDEVICE THAT NEED INITIALIZATION

The flHook flag must always be set with HOOK_TEXTOUT

Returns:  This function returns TRUE for success and FALSE for failure.

### 3.4.2.  FontDisableSurface ( )

This function is called by the Control module at DrvDisableSurface to allow the Font module to clean up its internal data and to deallocate any memory associated with the surface.

BOOL	FontDisableSurface(PDEVICE lpdv);

Returns: TRUE for success and FALSE for failure.

### 3.4.3.  FontDisablePDEV ( )

This function is called by the Control module at DrvDisablePDEV to allow the Font module to clean up its internal data and deallocate any memory associated with the PDEVICE.

BOOL	FontDisablePDEV(PDEVICE lpdv);

Returns: TRUE for success and FALSE for failure.

### 3.4.4 Font Functions

The following are possible export functions for the Vector module. The function prototypes for the exported Vector functions are the same as those equivalent DDIs defined in Win32 DDK.

FMTextOut ( ) - Required export.

FMQueryFont ( ) - Required export.

FMQueryFontTree ( ) - Required export.

FMQueryFontData ( ) - Required export.

FMFontManagement ( ) - Required export.

FMGetGlyphMode ( ) - Required export.

FMQueryAdvancedWidth ( ) - Required export.

## 3.5  DrvMemAlloc - DrvMemFree

The Control module will handle all resource allocation for the Raster/Vector/Font and OEM modules.  The Raster/Vector/Font and OEM modules must call DrvMemAlloc( ) and DrvMemFree for all memory allocation and deallocation.  The Control module will be responsible for releasing all memory allocated at DrvDisableDriver.

PVOID DrvMemAlloc ( HANDLE hDriver, DWORD dwFlags, DWORD dwSize);

HANDLE 	hDriver;					// Handle of the driver.

DWORD	dwFlags;				// Memory allocation flag such as ZEROINIT

DWORD	dwSize;					// Size of memory block to allocate in bytes.

The Control module will call EngAllocMem to allocate the memory.

Returns: Pointer to memory block requested or NULL for failure.

BOOL  DrvMemFree (HANDLE hDriver, PVOID pMemory);

HANDLE	hDriver;					// Handle of driver

PVOID		pMemory;				// Pointer to memory block to free

The Control module will call EngFreeMem to free the memory.

Returns: TRUE for success and FALSE for failure.

## 3.6	INTERFACE WITH OEM CUSTOM UI

Please refer to NTGDI\PRINTERS\CUSTOM\OEMCUST.DOC for details on how OEM customization is supported by the Control Module.

# 4.  PRINTING MODE

## 4.1  Vector vs. Raster Mode

Internally, Unidrv can operate in either vector mode or raster mode.  In vector mode, Unidrv would attempt to map graphics drawings directly to printer vector commands which is generally more efficient.  In raster mode, Unidrv would rasterize all graphics drawings into the banding buffer together with bitmaps.  Both modes can have device font support.

The printing mode available will depend on the type of devices.  For inkjets and dot-matrix and low-end laserjet devices, the only printing mode available is Raster mode.  For other printers, Vector mode and Raster mode will be supported.

The default printing mode is specified in the GPD.  The user will be allowed to select the print mode (if appropriate), once the selection is made, the sFlags field in DRIVEREXTRA will be updated to reflect the settings - DXFLAGS_VECTOR is set when vector mode is selected, otherwise it’s cleared.  At DrvEnablePDEV , the Control module will initialize the  PUDPDEV->dwFlags appropriately to reflect the printing mode selected.  If FLAGS_VECTOR is set, the print mode selected is vector.  Otherwise, it’s raster.

The following figure illustrates the determination whether to print the job in Vector mode or Raster mode.

The following figure illustrates the routing of DDI calls.

## 4.2  Banding vs. Non-Banding Mode

The printing mode also affects the Control module’s decision to use banding vs. non-banding.  In general, if the printing mode selected is vector,  the Control module will be in full page banding mode and device managed surface.  Otherwise, the Control module  will be in banding mode and GDI managed surface.  This means that the Control module calls GDI to create and manage the band buffer.

The following figure illustrates the determination of printing in banding or non-banding mode.

In the case of device managed surface, the Control module calls EngCreateDeviceSurface( ) where the size is the size of the page.  In the case of banding mode, the Control module determines the size of the band by calling EngCreateBitmap ( ) until it’s successful, decreasing the size of the band as necessary for each iteration.  Afterward, it calls EngMarkBandingSurface ( ) to mark the banding area.

In full page banding mode (vector print mode), the surface is the surface handle returned from EngCreateDeviceSurface.  In banding mode (raster print mode), the surface is the bitmap handle returned from EngCreateBitmap.

# 5.  INTERNAL BINARY DATA

The GPD parser parses the GPD text file into a binary data shared between the UI module and the Control module.  UIINFO is the structure containing all information related to handling device querying and displaying of graphical user interface.  DRIVERINFO is the rendering info struct.  It contains all the information and commands necessary to render the page and send the job down to the device.

The binary data file will be in a format similar to the one listed below.

BINARY FILE

INFOHEADER	infoheader;		// header for binary file, contains offsets

UIINFO		uiinfo;			// UI information

DRIVERINFO	driverinfo;		// render information

BYTE[ ]		heap;			// heap for binary data

…

## 5.1   INFOHEADER

This is the header for the binary file supported by GPD parser.  The header contains offsets to UIINFO and DRIVERINFO as well as version information.  All heap related data are kept in contiguous memory.  Note: All offsets are relative to the beginning of the binary data.  The Control Module will call AN UNDEFINED PARSER FUNCTION to get a pointer to INFOHEADER.  It uses this pointer to access DRIVERINFO information via. the INFOHEADER.dwDriverOffset field.

typedef struct _INFOHEADER {

DWORD	dwSize;			// sizeof(INFOHEADER)

DWORD	dwFileSize;		// sizeof BINARY DATA FILE

DWORD	dwVersion;		// parser’s version

DWORD	dwCheckSum;		// 32 bit CRC checksum of ASCII

// text file GPD/PPD.

DWORD	dwSignature;		// DRIVER_SIGNATURE_POSTSCRIPT

// DRIVER_SIGNATURE_UNIDRV

DWORD	dwUIInfoOffset;		// byte offset to UIINFO struct

DWORD	dwDriverOffset;		// byte offset to DRIVERINFO struct

} INFOHEADER, *PINFOHEADER;

## 5.2  DRIVERINFO

DRIVERINFO is the structure containing all information required to render the page and send commands down the printer devices.

typedef struct _ARRAYREF{

DWORD	dwCount;		// Number of items in the array

DWORD	dwOffset;		// byte offset to the array

// If the dwOffset is NULL, the ARRAYREF is invalid.

} ARRAYREF, *PARRAYREF

typedef struct _DRIVERINFO {

DWORD	dwSize;			// sizeof (DRIVERINFO)

ARRAYREF	Cmds;			// array reference to the COMMAND array

ARRAYREF	OrderDependency;	// array reference to the ORDERDEPENDENCY array

ARRAYREF	Parameter;		// array reference to the PARAMETER array

ARRAYREF	ValueStack;		// array reference to the value stack array

ARRAYREF	OperatorStack;		// array reference to the operator stack array

ARRAYREF	DWORDList;		// array reference to the LIST of DWORDs array

DWORD	dwGlobalOffset;		// byte offset to the global data

DWORD	dwStateVariableOffset;	// byte offset to the state variables table

DWORD	dwJobSetupIndex;	// byte offset to the JOBSETUP sequence cmd index

DWORD	dwDocSetupIndex;	// byte offset to the DOCSETUP sequence cmd index

DWORD	dwPageSetupIndex;	// byte offset to the PAGESETUP sequence cmd index

DWORD	dwPageFinishIndex;	// byte offset to the PAGEFINISH sequence cmd index

DWORD	dwDocFinishIndex;	// byte offset to the DOCFINISH sequence cmd index

DWORD	dwJobFinishIndex;	// byte offset to the JOBFINISH sequence cmd index

} DRIVERINFO, *PDRIVERINFO;

## 5.3   COMMAND

All commands listed in the GPD file will be parsed into the format listed below and stored in the Cmds array.  The command structure is arranged as a one dimensional array, accessible through a list of pre-defined index.  The invocation string can be in the form of  one or more binary string concatenated together.  Between binary string, there can exist parameter reference, always headed by a %.  For example,  %paramref, where paramref is the index into the Parameter array.  The value at the index specified is the parameter.

typedef struct _COMMAND{

INVOCATION		invocation;		// binary string(s) and parameter references .

// “binary string” %paramref ”binarystring”

ORDERDEPENDENCY	order;			// contains the orderdependency info

// for a list of commands.

DWORD		dwCmdCallbackID;	// Command callback ID defined in GPD.

// dwCmdCallbackID = 0xFFFFFFFF means

// the minidriver did not want to hook this cmd.

} COMMAND, *PCOMMAND;

For every command, the Control module will look at the dwCmdCallbackID, if it’s 0xFFFFFFFF, it’s treated as a regular command.  Otherwise, it is treated as a CmdCallback.  A command callback might required parameters.  In this case, the invocation string will contain indices to the state variable table in the order of the expected parameter where the indices are separated by a space.  The state variable table is defined below.

## 5.4  PARAMETER

The following defines the PARAMETER structure.  All the parameters required by the defined commands are stored in the Parameter array in DRIVERINFO.

typedef struct _PARAMETER{

BYTE		bFormat;			// specifies the format of the parameter.

// enumeration of the format is defined in the GPD.

BYTE 		bDigits;				// specifies the number of digits to be emitted

// this is valid only if the format is ‘d’ or ‘D’

DWORD	dwFlags;			// possible flag set are

// PARAMETER_MINUSED

// PARAMETER_MAXUSED

// PARAMETER_MAXREPEATUSED

LONG		lMin;				// specifies the min value

LONG 		lMax;				// specifies the max value

DWORD	lMaxRepeat;			// specifies the max repeat count

ARRAYREF	ValueStack;			// array reference to the ValueStack array.

ARRAYREF        OperatorStack;			// array reference to the OperatorStack array.

}PARAMETER, *PPARAMETER;

## 5.5  ValueStack/Operator Stack Array

The ValueStack array is an array of VALUE structures.  The OperatorStack array is an array of WORDs.  Each item in the array contains predefined operator. The operators are defined in GPD specification.

typedef struct _VALUE{

DWORD	dwFlags;			// VALUE_SYMBOL.

// If the VALUE_SYMBOL bit is set, the Control

// module will assume the value is an index to the

// state variable table. Otherwise, dwValue is the

// actual value.

DWORD	dwValue;			// the value

} VALUE, *PVALUE;

In the case where the PARAMETER specifies an operator stack and a value stack, the Control module uses Reverse Polish to calculate the parameter for the command.  The operator stack would be used until it reached the HALT operator.  At this time, the value on top of the value stack is formatted according to the format specified and emitted.

## 5.6 SEQUENCE COMMAND

The OrderDependency list is an array of ORDERDEPENDENCY structure.  The OrderDependency list in the binary data is grouped into 6 different sections.  The index to these sections are defined in DRIVERINFO.  The following sections are defined - Job_Setup, Doc_Setup, Page_Setup, Page_Finish, Doc_Finish, Job_Finish.

typedef struct _SEQUENCE{

WORD	wIndexOfCommand;			// index into command array

WORD	wFeatureIndex;				// index into feature array

WORD	wNextInSequence;			// next  in list.

// 0xFFFF means the end of the sequence.

WORD	wReserved;				// reserved.

} SEQUENCE, *PSEQUENCE;

## 5.7 ORDERDEPENDENCY

Orderdependency will not be used by the Control module, it’s present as a reference for the parsers.

typedef struct _ORDERDEPENDENCY {

WORD		section;				// specifies which section.

WORD 		order;				// specifies the order value.

} ORDERDEPENDENCY; *PORDERDEPENDENCY;

## 5.8  STATE VARIABLES

The Control module will maintain a state variable table used for specifying parameters for commands.  In general, if the VALUE_SYMBOL bit is set in a value structure on the value stack, the parameter is an index reference to the state variable table.  A list of state variables are listed here.  For more information on the state variables, refer to the GPD File Format Specification.  The indices assignment are TBD.

NumOfDataBytes - number of data bytes.

RasterDataWidthInBytes - width of scanlines in bytes.

RasterDataHeightInPixels - height of raster data block in pixels.

NumOfCopies - number of copies requested by the user.

PrintDirInCCDegrees - requested rotation degree.

X_cursor - X cursor position.

Y_cusor - Y cursor position.

RelX - X cursor position relative to current X cursor position.

RelY - Y cursor position relative to current Y cursor position.

LinefeedSpacing - vertical spacing a linefeed generates.

RectXSize - width of the rectangle.

GrayPercentage - the gray level in percentage.

NextFontID - ID of softfont to be download next.

NextChar - next character to be printed next.

PhysPaperLength - length of physical paper.

PhysPaperWidth - width of physical paper.

FontHeight - height of font.

FontWidth - width of fixed pitch font.

FontIntLeading - internal leading of font.

TextYRes - the current text Y resolution.

TextXRes - the current text X resolution.

GraphicsXRes - the current X graphic resolution

GraphicsYRes- the current Y graphic resolution

## 5.9  GLOBAL ATTRIBUTES AND COMMANDS

Global attributes are information that is needed to initializes the PDEVICE.  Global commands are general printing commands.  The following is a list of all currently known global attributes.  There will be indices assigned to each of the items below.  The indices assignment are TBD.

For more information, refer to the GPD specification.

### 5.9.1 GLOBAL ATTRIBUTES

GPDSpecVersion - the GPD spec. version.

MasterUnits - the X and Y master units.

Include - name of GPD file to be embedded.

ResourceDLL - the name of the resource dll.

ModelName - printer model name.

rcModelNameID - resource string ID for the printer model name.

PrinterType - specifies the printer type.

MaxCopies - maximum number of copies supported.

MaxPrintableArea - maximum printable width and height of a printer.

MinLeftMargin - minimum left margin of a printer.

OutputDataFormat - output data direction.

ChangeColorModeOnPage - indicates whether the print allows changing color mode on a per page basis.

DefaultFont - the name of the default font.

The following information are the GPD.  They might not be in the same organization as defined below.  It’s more efficient for the parser to initialize these flags for the Control module at parsing time.

sBitsPixel - bits per pixel for driver’s bitmap rendering buffer. From COLORMODE.

sPrinterPlanes - number of planes to send to printer.

sColorPlaneOrder - the order of sending color plane data.

cMaxSoftFont - specifies the maximum number of soft fonts.

sMaxGray - maximum value for gray level.

sMinGray - minimum value for gray level.

sLookAhead - size of lookahead region to use in controlling ordering of text/graphics output for inkjet printers.

sMaxPageFonts - maximum number of fonts this printer model can place on a single page.

fCursorXMove - flags describing the X movement capabilities.

FCURSORXMOVE_NO_POR_GRX

FCURSORXMOVE_NO_LAN_GRX

FCURSORXMOVE_RESET_FONT

FCURSORXMOVE_FAV_ABS

FCURSORXMOVE_REL_LEFT

FCURSORXMOVE_ABS_NO_LEFT

FCURSORXMOVE_RES_DEPENDENT

fCursorYMode - flags describing the Y movement capabilites.

FCURSORYMOVE_NO_POR_GRX

FCURSORYMOVE_NO_LAN_GRX

FCURSORYMOVE_CR

FCURSORYMOVE_SETLINESSPACING

FCURSORYMOVE_TRUNCATE

FCURSORYMOVE_FAV_ABS

FCURSORYMOVE_REL_UP

fCompressMode - specifies the compression mode available.  The parser get this information from Cmds.

COMPRESS_RLE

COMPRESS_TIFF40

COMPRESS_DELTAROW

COMPRESS_BITREPEAT

COMPRESS_FE_RLE

fRectFill - Bit flags describing the fill rectangle capabilities.

FRECTFILL_CUR_X_END

FRECTFILL_CUR_Y_END

fGeneral - Bit flags describing the general printer capabilities.

FGENERAL_SERIAL

FGENERAL_ROTATE_FONT_ABLE

FGENERAL_COPIES

FGENERAL_DUPLEX

FGENERAL_NO_ADJACENT

FGENERAL_LANDSCAPE_GRX_ABLE

FGENERAL_ALIGN_BASEMENT

FGENERAL_FONT_MEMCFG

FGENERAL_LANDSCAPE_RT90

FGENERAL_USE_CURSOR_ORIG

FGENERAL_WHITE_TEXT

FGENERAL_PCL_PAGEPROTECT

FGENERAL_MARGINES

FGENERAL_CMD_CALLBACK

FGENERAL_MEMRES

fText - Bits flags describing the devices hardware font capabilities

FTEXT_RA_ABLE

FTEXT_CR_90

FTEXT_VA_ABLE

fCurves - Bits flag describing curve capabilities

FCURVES_CIRCLES

FCURVES_PIE

FCURVES_CHORD

FCURVES_ELLIPSES

FCURVES_WIDE

FCURVES_STYLED

FCURVES_WIDESTYLED

FCURVES_INTERIORS

FCURVES_ROUNDRECT

FCURVES_POLYBEZIER

fLines - Bits flag describing line capabilities

FLINES_POLYLINE

FLINES_MARKER

FLINES_POLYMARKER

FLINES_WIDE

FLINES_WIDESTYLED

FLINES_INTERIORS

fPolygonal - Bits flag describing polygonal capabilities

FPOLYGONAL_ALTPOLYGON

FPOLYGONAL_RECTANGLE

FPOLYGONAL_WINDPOLYGON

FPOLYGONAL_SCANLINE

FPOLYGONAl_WIDE

FPOLYGONAL_STYLED

FPOLYGONAL_WIDESTYLED

FPOLYGONAL_INTERIORS

FPOLYGONAL_POLYPOLYGON

fClip - Bits flags describing clip capabilities

FCLIP_NONE

FCLIP_RECTANGLE

### 9.5.2 GLOBAL COMMANDS

GENERAL PRINTING COMMANDS

CmdCopies - command to set the count for multiple-copy printing.

CmdSleepTimeOut - command to set the number of minutes in idle mode before the printer goes into sleep mode.

CmdSetLineSpacing - command to set the line spacing.

CmdUniDirectionOn - command to enable unidirectional printing.

CmdUniDirectionOff  - command to disable unidirectional printing.

CmdPushCursor - command to push the cursor position on the stack.

CmdPopCursor - command to pop the cursor position from the stack.

CmdBackSpace - command to backspace a single character width.

CmdBeginPaletteDef - command to initialize color palette definition.

CmdEndPaletteDef - command to finish color palette definition.

CmdDefinePaletteEntry - command to define a color palette entry.

CmdSelectPaletteEntry - command to select a color palette entry as the current color.

CmdSetSimpleRotation - command to set the rotation angle in multiples of 90 degress cc.

CmdSetAnyRotation - command to set the rotation angle in cc direction.

CmdSetOverlayID - command to set the ID for the overlay pattern to be downloaded.

CmdSelectOverlayID - command to select the ID of a downloaded overlay pattern.

CmdSendBlockData - command to send one block of data to the printer.

CURSOR MOVE COMMANDS

CmdXMoveAbsolute - command to move the cursor in to absolute X position.

CmdXMoveRelLeft - command to move the cursor relatively left X position.

CmdXMoveRelRight - command to move the cursor relative right X position.

CmdYMoveAbsolute - command to move the cursor in to absolute Y position.

CmdYMoveRelUp - command to move the cursor relatively up Y position.

CmdYMoveRelDown - command to move the cursor relative down Y position.

CmdXYMoveAbsolute - command to move the cursor to absolute X, Y position.

RASTER PRINTING COMMANDS

CmdBeginRaster - command to initialize raster data transfer.

CmdEndRaster - command to finish raster data transfer.

CmdSetDestBmpWidth - command to set the desitnation bitmap width.

CmdSetDesBmpHeight - command to set the destination bitmap height.

CmdSetSrcBmpWidth - command to set the src bitmap width

CmdSetSrcBmpHeight - command to set the src bitmap height.

CmdEnableTIFF4 - command to enable TIFF4 compression.

CmdEnableDRC - command to enable DRC.

CmdDisableCompression - command to disable printer compression.

CmdSendYellowData - command to send Y plane data.

CmdSendMagentaData - command to send M plane data.

CmdSendCyanData - command to send C plane data.

CmdSendBlackData - command to send K plane data.

CmdSendRedData - command to send R plane data

CmdSendGreenData - command to send G plane data

CmdSendBlueData - command to send B plane data.

CmdSelectYellowColor - command to select yellow color.

CmdSelectMagentaColor - command to select magenta color.

CmdSelectCyanColor - command to select cyan color.

CmdSelectBlackColor - command to select black color.

CmdSelectRedColor - command to select red color.

CmdSelectGreenColor - command to select green color.

CmdSelectBlueColor - command to select blue color.

TEXT PRINTING COMMANDS

CmdSelect - command to slect the font.

CmdUnSelect - command to unselect the font

CmdBoldOn - command to bold of subsequent characters.

CmdBoldOff - command to disable bolding of subsequent characters.

CmdItalicOn - command to italicized subsequent characters.

CmdItalicOff - command to disable italicizing of subsequent characters.

CmdUnderlineOn - command to underline subsequent characters.

CmdUnderlineOff - command to disable underlining of subsequent characters.

CmdStrikeThruOn - command to strikethrough subsequent characters

CmdStrikeThruOff - command to disable stikethrough of subsequent characters

CmdWhiteTextOn - command to begin white text.

CmdWhiteTextOff - command to end white text.

CmdSelectSingleByteMode - command to end KANJI mode.

CmdSelectDoubleByteMode - command to start KANJI mode.

CmdVerticalPrintingOn - command to start KANJI vertical mode.

CmdVerticalPrintingOff - command to end KANJI vertical mode.

CmdSetFontID - command to set an ID for a font to be downloaded.

CmdSelectFontID - command to select and ID for a downloaded font.

CmdSetCharCode - command to set the character code.

CmdSetTCBlack - command to set text color to black.

CmdSetTCRed - command to set text color to red.

CmdSetTCGreen - command to set text color to green.

CmdSetTCYellow - command to set text color to yellow.

CmdSetTCBlue - command to set text color to blue.

CmdSetTCMagenta - command to set text color to magenta.

CmdSetTCCyan - command to set text color to cyan.

CmdSetTCWhite - command to set text color to white.

VECTOR PRINTING COMMANDS

CmdSetRectWidth - command to set the rectangle width.

CmdSetRectHeight - command to set the rectangle height.

CmdSetRectSize - command to set the rectangle width and height.

CmdRectGrayFill - command to perform gray fill on a rectangle.

CmdRectWhiteFill - command to perform white fill on a rectangle.

CmdRectBlackFill - command to perform black fill on a rectangle.

## 5.10 RENDERING INFORMATION FOR PREDEFINED FEATURES

Predefined features are features that the driver and the GPD parser are aware of through predefined GIDs.  Each predefined feature has a unique option structure that describes the UI information.  In addition, the option structure contains a pointer to render specific information.

This section will define the render specific information for each predefined feature.  In general, the dwRenderOffset pointer defined in the UI module point to the corresponding render information listed below.  For more information, refer to the UI Module Design Specification.

GID_RESOLUTION

typedef struct _RESOLUTIONEX{

POINT		TextDPI;		// X and Y units of text resolution.

WORD		wPinsPerPhysPass;	// number of scanlines printed when printhead moves across

// page.

WORD		wPinsPerLogPass;	// number of scanlines printed in one logical pass

WORD		wDefaultDithering;	// specifies the default dithering method.

WORD		wSpotDiameter;		// the spot diameter value.

POINT		TextResolution;		// the X and Y units of text resolution.

WORD		wFlag;			// SUPPORT_COLOR - color is supported.

WORD		wReserved;

} RESOLUTIONEX; *PRESOLUTIONEX;

GID_COLORMODE

typedef struct COLORMODEEX{

WORD		wPrinterNumofPlanes;	// the number of color planes for color data sent to printer.

WORD		wPrinterBPP;		// number of bits per pixel for color data sent to printer.

WORD		wColorPlaneOrder;	// the order of sending color plane raster data.

WORD		wFlag;			// COLOR_SEPARATION - require color separation.

// USE_COLORSELECTCMD - color selection cmd

// are separate from the cmds to send color raster data.

// MOVE_TOX0BEFORESELECTCMD - moving

// to physical x=0 before sending the color selection cmd.

} COLORMODEEX; *PCOLORMODEEX;

GID_PAGESIZE

typedef struct _PAGESIZE {

RECTL		rcMargins;		// the margins of the papersize in master units

POINT		CursorPosition;		// offset of the cursor relative to the upper left corner

// of the logical page.

POINT		VectorOffset;		// the offset of vector relative to the upper left corner

// of the logical page.

POINT		MinSize;		// the minimum X and Y dimensions for the CUSTOMSIZE

POINT		MaxSize;		// the maximum X and Y dimensions for CUSTOMSIZE options

} PAGESIZEEX; *PAGESIZEEX;

GID_INPUTSLOT

typedef struct _INPUTSLOTEX {

POINT		FeedMargins;		// The margins for the paper feed option.

WORD		wFlag;			// FACEUP_NONE, FACEUP_CENTER, FACEUP_LEFT

// FACEUP_RIGHT.

// FACEDOWN_NONE, FACEDOWN_CENTER,

// FACEDOWN_LEFT, FACEDOWN_RIGHT

} INPUTSLOTEX; *PINPUTSLOTEX;

GID_CUSTOMSIZE - no extra information is needed for rendering.

GID_DUPLEX - no extra information is needed for rendering.

GID_COLLATE - no extra information is needed for rendering.

GID_PAGEPROTECTION - no extra information is needed for rendering.

GID_COPIES - no extra information is needed for rendering.

GID_ORIENTATION - no extra information is needed for rendering.

GID_OUTPUTBIN - no extra information is needed for rendering.

GID_MEDIATYPE - no extra information is needed for rendering.

GID_MEMOPTION - no extra information is needed for rendering.

# 6.   DEVINFO

The DEVINFO struct is used by DrvEnablePDEV function to describe the driver and the PDEV.  This structure is passed into the driver at DrvEnablePDEV.  The Control module will call FontInit( ) and  either VectorInit( ) or RasterInit( ) and to initialize DEVINFO at DrvEnablePDEV time.

typedef struct DEVINFO {

FLONG flGraphicsCaps;

LOGFONTW lfDefaultFont;

LOGFONTW lfAnsiVarFont;

LOGFONTW lfAnsiFixFont;

ULONG cFonts;

ULONG iDitherFormat;

USHORT cxDither;

USHORT cyDither;

HPALETTE hpalDefault;

} DEVINFO, *PDEVINFO;

flGraphicsCaps - Flags used to describe graphics capabilities of the driver.

Vector Mode - flGraphicsCaps = GCAPS_BEZIERS | GCAPS_GEOMETRICWIDE | GCAPS_ALTERNATEFILL | GCAPS_WINGDINGFILL | GCAPS_HALFTONE | GCAPS_COLOR_DITHER | GCAPS_MONO_DITHER | GCAPS_ARBRUSHOPAQUE | GCAPS_VECTORFONT | GCAPS_OPAQUERECT |GCAPS_FORCEDITHER(only if the printer is a color device).

Raster Mode - flGraphicscaps = GCAPS_HALFTONE | GCAPS_COLOR_DITHER | GCAPS_MONO_DITHER | GCAPS_ARBRUSHOPAQUE | GCAPS_FORCEDITHER (only if the printer is a color device)

This field is initalized by either VectorInit ( ) or RasterInit ( ).

lfDefaultFont - An extended logical font structure that specifies the default font for a device.

This field is initialized by the Font module’s FontInit( ) function.

lfAnsiVarFont - An extended logical font structure that specifies the default variable-pitch font for a device.

This field is initialized by the Font module’s FontInit( ) function.

lfAnsiFixFont - An extended logical font structure that specifies the default fixed-pitch font for a device.

This field is initialized by the Font module’s FontInit( ) function.

cFonts - Specifies the number of device fonts.

This field is initialized by the Font module’s FontInit( ) function.

iDitherFormat - Specifies the format of the bitmap.  The format is defined in the GPD file and is stored in the binary data structure.  The Control Module reads this information from the binary data and initializes iDitherFormat as appropriate.  The followings are all the possible format

BMF_1BPP	Monochrome.

BMF_4BPP	4 bits per pixel.

BMF_8BPP	8 bits per pixel.

BMF_16BPP	16 bits per pixel.

BMF_24BPP	24 bits per pixel.

BMF_32BPP	32 bits per pixel.

BMF_4RLE	4 bits per pixel, run length encoded.

BMF_8RLE	8 bits per pixel, run length encoded.

cxDither and cyDither - Specifies the dimensions of a dithered brush.  These values would come from the GDP if non-printer dithering is selected.

hpalDefault - Specifies the default palette for the device.  The Control Module calls EngCreatePalette to create the default palette, PAL_RGB is used for 24 bit devices and PAL_INDEXED is used for all others.

# 7.  GDIINFO

The GDIINFO strucutre describes the graphics capabilities of a given device.  GDI zero-initializes this structure before calling the driver’s DrvEnablePDEV function.  The Control module will call FontInit ( ) and either VectorInit( ) or RasterInit ( ) to fill out the GDIINFO.  The Control module expects those modules to fill out only the fields marked below.  The Control module will fill out the rest of the fields.

typedef struct _GDIINFO {

ULONG ulVersion;				// HIWORD is set to zero.  LOBYTE of LOWORD is the OS

// version, HIBYTE of LOWORD is the driver version.

// Control module fills this field.

ULONG ulTechnology;			// Set to DT_RASPRINTER.  Control module fills this field.

ULONG ulHorzSize;				// The width of the physical surface in mm.  This value is

// obtained from the current paper selected.

// Control module fills this field.

ULONG ulVertSize;				// The height of the physical surface in mm.  This value is

// obtained from the current paper selected.

// Control module fills this field.

ULONG ulHorzRes;				// The width of the physical surface in pixel.

// The Control module fills this field.

ULONG ulVertRes;				// The height of the physical surface in pixel.

// The Control module fills this field.

ULONG cBitsPixel;				// The BPP as defined in *DrvBPP in COLORMODE.

// The Control module fills this field.

ULONG cPlanes;				// The number of planes as defined in COLORMODE.

// The Control module fills this field.

ULONG ulNumColors;			// The number of physical color supported by device.

// The Control module fills this field.

ULONG flRaster;				// Reserved, must be zero.

ULONG ulLogPixelsX;			// The horizontal logical pixels per inch.

// The Control module fills this field.

ULONG ulLogPixelsY;			// The vertical logical pixels per inch.

// The Control module fills this field.

ULONG flTextCaps;				// Flag specifies the text capabilities.

// The Font module’s FontInit ( ) function fills this field.

ULONG ulDACRed;				// Specifies the display number of DAC bits.

ULONG ulDACGreen;			// These three fields are not set by the Control module

ULONG ulDACBlue;				// They don’t applied to our driver.

ULONG ulAspectX;				// The relative width and height of a device pixel.

ULONG ulAspectY;				// This is the same as X and Y resolution.

ULONG ulAspectXY;				// The hypothenuse of the X and Y resolution.

// The Control module fills these fields.

LONG xStyleStep;				// The numerator of style advace of x and y major lines.

LONG yStyleStep;				// These values are set to 1.

LONG denStyleStep;				// The denominator of style advance.

// The Control module fills these fields.

POINTL ptlPhysOffset;			// The size in pixels of the margin of the surface.

SIZEL szlPhysSize;				// The size in  pixels of the entire surface, including margins.

// The Control module fills these fields.

ULONG ulNumPalReg;			// The number of registered palette.  This field is not filled.

COLORINFO ciDevice;			// The COLORINFO struct that defines the device’s colors in

// CIE coordinate space.  The Control module fills this field

// from DEVHTINFO stored in registry.

ULONG ulDevicePelsDPI;			// Specifies the number of pixels per inch  if the pixels are laid

// out side by side without overlapping or space between.  The

// Control module fills this field with info from DEVHTINFO.

ULONG ulPrimaryOrder;			// Specifies the bit order of the device’s primary colors or plane

// numbers for the halftone output.

// What should the control module set this field to?

ULONG ulHTPatternSize;			// Specifies the size of the halftone pattern.  The Control module

// fills this field from DEVHTINFO stored in registry.

ULONG ulHTOutputFormat;			// Specifies the output format for halftone.  The Control module

// fills this field from DEVHTINFO stored in registry.

ULONG flHTFlags;				// Specifes the flags describing the halftoning of the device.  The

// Control module fills this field from DEVHTINFO.

ULONG ulVRefresh;				// These fields are not set by the Control module since they

ULONG ulBltAlignment;			// don’t applied to our driver.

ULONG ulPanningHorzRes;

ULONG ulPanningVertRes;

} GDIINFO, *PGDIINFO;

# 8.	PDEVICE

The PDEVICE is private to the driver and contains all the information and data associated with the physical device.  At DrvEnablePDev ( ) , the driver setup the following structure using the data from the internal binary data as well as calling the Raster/Vector/Font/OEM modules to allow it to initialize its own data structure in the PDEVICE.

## 8.1  UNIDRV PDEVICE

typedef  struct

{

// PUBLIC INFORMATION FOR OEM …….

MDEV		*pMDev;		/* Minidriver PDEVICE		*/

// PRIVATE INFORMATION FOR OEM …..

// For more details, refer to the OEM customization specification .

PVOID		pOEMDevmode;		/* Pointer to OEM DEVMODE data */

PDRIVERFNS	pDriverFns;		/* Driver helper functions */

DRIVERNEABLEDATA pDed;		/* Driver DDI table for OEM to initalize */

// GENERAL  INFORMATION	…….

ULONG    	ulID;               		/* String to verify that we have our PDEV */

HDEV     	hdev;               		/* Handle for the device.Use this handle when calling GDI back */

HANDLE   	hPrinter;           		/* For access to spooler data, WritePrinter( ) */

HBITMAP  	hbm;                		/* Handle to GDI managed bitmap created at DrvEnableSurface */

HSURF		hsurf;			/* Handle to device managed surfaced created at

DrvEnableSurface*/

PWSTR    	pstrPrtName;        	/* Printer name used in CreateDC() */

PWSTR		pstrDataFile;		/* Name of minidriver */

PWSTR    	pstrModel;         	 	/* Model name of printer, e.g LaserJet II */

void		*pInfoHeader;		/* Pointer to INFOHEADER struct */

void		*pDriverInfo;		/* Pointer to DRIVERINFO struct*/

void      		*pUIInfo;		/* Pointer to UIINFO struct */

void       		*pso;                		/* For EngCheckAbort() */

HANDLE	hMiniHandle;		/* Handle to minidriver module */

DWORD	fMode;			/* Device context flags */

DWORD	fFlags;			/* PDEVICE flags.  VECTOR/RASTER etc. */

short       	sCopies;            		/* # of copies requested. */

short		sPaperSource;		/* Paper source index */

PAPERFORMAT pfPaper;		/* Current paper size format structure */

// BIT FLAGS

DWORD	dwFlags;		/* General flag to keep track of settings */

/* 	FLAG_PRINTMODE_VECTOR, not set = RASTER

FLAG_DUPLEX_DUPLEX, not set = SIMPLEX

FLAG_COLOR_COLOR, not set = MONOCHROME

// MEMORY MANAGEMENT INFORMATION ……

MEMLINK	*lpMemLinkList;		/* Pointer to a linked list of handles/pointers to memory allocated

for Raster/Font/Vector/OEM modules */

DWORD	dwMemAvailable;	/* Bytes of usable memory in printer */

DWORD	dwTotalPrinterMem;	/* Bytes of total memory in printer */

DWORD	dwFontMem;		/* Bytes of allocated printer memory for font download */

DWORD	dwFontMemUsed;	/* Bytes of printer memory used for downloaded fonts */

// TEXT SPECIFIC INFORMATION 	…….

DWORD       	fSelBits;          		/* Font selection bits */

void    		*pPSHeader;          	/* Position sorting header (posnsort.[hc]) */

void    		*pvWhiteText;        	/* White text structures, if needed */

short       	sDefCTT;            		/* Default translation table */

int         		iUsedSoftFonts;  		/* Number of soft fonts used */

int         		iNextSFIndex;       	/* Index ID to use for next softfont */

int         		iFirstSFIndex;      	/* Value used to reset the above */

int         		iLastSFIndex;       	/* Largest value available */

POINTW      	ptDefaultFont;      	/* default font width & height. */

FONTMAP    	*pFontMap;           	/* Array of FONTMAPS describing fonts. */

FONTMAP    	*pFMDefault;         	/* Default font FONTMAP,  if != 0 */

FONTMAP    	*pFMCurDL;           	/* Current font download ID - pseudo inc */

int         		cFonts;             		/* Number of FONTMAPS in the above */

int         		cBIFonts;           		/* Number of device fonts: no soft fonts */

void       		*pvFIMem;            	/* Installed fonts access data */

void       		*pvDLMap;            	/* Mapping of GDI to downloaded info */

int         		iCurXFont;          		/* Index of currently selected font */

DWORD      	*pdwFontAvail;       	/* Bit array: available minidriver fonts */

int         		iMaxDevFonts;       	/* Highest index in the above array */

PDH         	pdh;                		/* long pointer to the start of tables. */

// GRAPHICS SPECIFIC INFORMATION ………

HPALETTE 	hpal;              	 	/* Handle to our palette created at DrvEnablePDEV */

DWORD   	*pdwTrans;           	/* Transpose table,  use for transposing bitmap , if required */

DWORD   	*pdwColrSep;         	/* Colour separation data, if required */

DWORD   	*pdwBitMask;         	/* Bitmask table,  white skip code */

void    		*pPalData;           	/* Palette information */

COLORADJUSTMENT  ca;            	/* For halftone/stretchblt code */

int         		ixgRes;             		/* Resolution, x graphics resolution*/

int         		iygRes;             		/* Resolution, y graphics resolution */

// OUTPUT CONTROL INFORMATION  ……….

OUTPUTCTL	ctl;                		/* state of the printer */

RECTL       	rcClipRgn;          		/* only draw to this clipping rectangle */

SIZEL       	szlPage;            		/* Whole page, in graphics units */

SIZEL       	szlBand;            		/* Size of banding region, if banding   */

int         		iBandDirection;     	/* Direction of banding                 */

BOOL        	bBanding;           		/* Used to indicate we are banding      */

// RENDERING INFORMATION ……

void    		*pvRenderData;       	/* Rendering summary data */

void    		*pvRenderDataTmp;    	/* Temporary copy for use in banding */

void    		*pRuleData;          	/* Rule finding data area, if != 0 */

// VECTOR INFORMATION ……….

void		*pvVectorPDEV;		/* Vector device PDEV, LPVDV	*/

// ANALYSIS INFORMATION …..

WORD		fPreAnalysis;		/* Pre-analysis flag */

ANALYSIS     	AnalysisInfo;		/* Pre-analysis info */

// SPOOLER INFORMATION ……

int		cbSpool;			/* Offset into the spool buffer */

BYTE		*pbSpoolBuf;		/* Output spool buffer base address */

}  PDEVICE;

# 8.  DEVMODE

The changes in DEVMODE only affect the DRIVEREXTRA part of the EXTDEVMODE.  There are some redefinition of fields and the removal of rgindex[ ] array.  In addition, the font cartridges information is considered printer-sticky thus have been moved to the registry.

The settings for the document is saved in the option[ ] array.  This array maps to the FEATURE list in the binary data.  The values saved in the array represent the option selections for the features.

## 8.1 PUBLIC DEVMODE

typedef struct _DEVMODE {

WCHAR	dmDeviceName[CCHDEVICENAME];

WORD	dmSpecVersion;

WORD   	dmDriverVersion;

WORD   	dmSize;

WORD   	dmDriverExtra;

DWORD  	dmFields;

short  	dmOrientation;

short  	dmPaperSize;

short  	dmPaperLength;

short  	dmPaperWidth;

short  	dmScale;

short  	dmCopies;

short  	dmDefaultSource;

short  	dmPrintQuality;

short  	dmColor;

short  	dmDuplex;

short  	dmYResolution;

short  	dmTTOption;

short  	dmCollate;

WCHAR  	dmFormName[CCHFORMNAME];

WORD   	dmLogPixels;

DWORD  	dmBitsPerPel;

DWORD  	dmPelsWidth;

DWORD  	dmPelsHeight;

DWORD  	dmDisplayFlags;

DWORD  	dmDisplayFrequency;

} DEVMODE;

## 8.2 PRIVATE DEVMODE

typedef struct _DRIVEREXTRA{

short	sVer;					// DRIVEREXTRA version

// dmSize was dmDefaultSource.

WORD			dmSize;			// size of DRIVEREXTRA

// dmOEMDriverExtra was dmTextQuality

WORD			dmOEMDriverExtra;	// total size of OEM DRIVEREXTRA

short			sMiniVer;		// GPC version

short			dmBrush;		// type of dithering brush (0)

short			sCTT;			// CTT index

short			reserved[36];		// reserved

// reserved[36] came from eliminating the following fields

//short			dmNumCarts;			// Not used, in registry

//short			rgFontCarts[MAXCART]	;	// Not used, in registry

//short			dmMemory;			// Not used, in PRINTERDATA

// Basically, rgindex [ ] is replaced by options [ ] array defined below.

//short			rgindex[MAXHE];		// Not used, in PDEVICE

short			sFlags;			// general flags

// DXF_NORULES (ON/OFF)

// DXF_TEXTASGRAPHICS

// DXF_JOBSEP

// DXF_PAGEPROT

// DXF_VECTOR

short			Padding;			// alignment

COLORADJUSTMENT	ca;			// system halftone information

DWORD		dwDriverSignature;	// DRIVER_SIGNATURE

DWORD		dwCheckSum32;		// 32 bit checksum

// wOptionCount is the number of doc-sticky and predefined features.

// options [ ] is an array of doc-sticky and predefined options

// MAX_DOC_OPTION is 256

WORD			wOptionCount;			// number of doc-sticky options	OPTSELECT		option [MAX_DOC_OPTION];	// array of doc-sticky and

// predefined options

} UNIDRIVEXTRA, *PUNIDRIVEXTRA;

typedef struct _EXTDEVMODE{

DEVMODE		dm;			// public devmode

DRIVEREXTRA		dx;			// driver devmode

} EXTDEVMODE, *PEXTDEVMODE;

# 9.	PRE-ANALYSIS

The goal of pre-analysis is to increase performance of printing and to use less resources.  The idea is that for a given job, printing to a color device, not all pages would be in color.  We can optimize on a per page basis.  For a monochrome page, we render it at 1bpp, for a color page, we render in the format selected.  Another advantage is that if we know what is on the page, where it’s on the page, we can skip blank areas on a page. These optimizations could reduce our printing time and our memory requirement, especially in the banding scenario.  In addition, we can use pre-analysis to prevent the z-ordering problems seen with device font and vector graphics when mixed with bitmaps .  Note that we can render the data in the most optimized format but the data send to the printer must be in the format specified by the GPD specification.  It is assumed that the Raster module will handle the conversion and the dump code.

## 9.1  GDI Requirements

Pre-analysis can be performed by either the Driver or GDI, as discussed in the sections below.  Either way, there are some GDI changes required in order to support rendering of different format within a job and detection of z-ordering problems.

GDI requirements:

- GDI must support multiple surface formats within a job  GDI must allows the driver to change the surface format on a per band basis.

- GDI must allow the driver to change the hook flag of the EngAssociateSurface() call on a per band basis.

- GDI must allow the driver to change device and font capabilities on a per band basis.

## 9.2  GDI performs Pre-Analysis

GDI needs to perform the analysis  at EMF recording time.  The following is a list of information that GDI must report to the driver. ( This assumed that GDI would support all requirements listed above).

- Interface for Driver to notify that it wants pre-analysis.

- Interface between GDI and Driver to communicates the pre-analysis information.

- A linked list of bounding box of the objects on the page.

- Information on whether the page is color or monochrome.

- Information on whether z-ordering problem is detected.

## 9.3  Driver performs Pre-Analysis

The Driver would have to perform the analysis at EMF playback time.  The following sections outline the items that the driver must support to perform pre-analysis.  Pre-analysis  will be performed in vector, true color, full page banding mode.  Data collected at pre-analysis pass consist of three types: monochrome/color information, bounding boxes information, and z-ordering detection.

The following structure will be added to UDPDEV to accommodate pre-analysis.  This structure is used to record monochrome/color, bounding boxes and z-ordering information.

typedef struct _ANALYSIS {

DWORD	dwFlags;			// Flags to indicate what is detected.

// ANALYSIS_COLOR_DETECTED

// ANALYSIS_ZORDERING_DETECTED_ROP

// ANALYSIS_ZORDERING_DETECTED_FONTS

RECTL		rcCurrentDim;			// Current dimension of the band.

} ANALYSIS; *PANALYSIS;

### 9.3.1  Determination for enabling pre-analysis

Pre-analysis and the data collected during the analysis pass will vary depending on the color mode selected, print mode selected and the type of device.  An example of the color mode selection would be  allowing the user to select one of the four color mode: Monochrome, GrayScale, Color Text & Graphics, and Photographic.  The ColorMode feature is a standard feature defined in the GPD.  The name of each of the color mode is represented in the GPD ColorMode Option *Name entry.  Refer to GPD Specification for more details.

Dot Matrix - No pre-analysis performed.

Serial InkJet - Pre-analysis is performed only if DrvBPP (rendering format specified in GPD) is NOT 1 BPP.  Z-ordering detection is only enabled if the device supports device fonts or downloading TT fonts.

Laser - Pre-analysis is performed only if DrvBPP (rendering format specified in GPD) is NOT 1 BPP regardless of the print mode selected (raster or vector).  Z-ordering detection is only enabled if the device support device fonts , downloading TT fonts or vector print mode is selected.

In addition, collection of monochrome/color and bounding boxes information would be disabled under the following conditions:

- PUDPDEV->fColorFormat & DC_PRIMARY_RGB

A flag, fPreAnalysis will be added to UDPDEV of Unidrv.  The flag indicates whether pre-analysis should be performed and the types of data to be collected.

PREANALYSIS_ENABLED

PREANALYSIS_MONOCOLOR

PREANALYSIS_BOUNDINGBOXES

PREANALYSIS_ZORDERING

### 9.3.2 Report all capabilities to GDI

The following is a driver function table returned to GDI at DrvEnableDriver ( ).

DRVFN  DrvFnTab[ ] =

{

{INDEX_DrvEnablePDEV,				(PFN)DrvEnablePDEV},

{INDEX_DrvCompletePDEV, 				(PFN)DrvCompletePDEV},

{INDEX_DrvDisablePDEV, 				(PFN)DrvDisablePDEV},

{INDEX_DrvEnableSurface,				(PFN)DrvEnableSurface},

{INDEX_DrvDisableSurface,				(PFN)DrvDisableSurface},

{INDEX_DrvResetPDEV,					(PFN)DrvResetPDEV},

{INDEX_DrvRealizeBrush,				(PFN)DrvRealizeBrush},

{INDEX_DrvStrokePath,					(PFN)DrvStrokePath},

{INDEX_DrvFillPath,					(PFN)DrvFillPath},

{INDEX_DrvStrokeAndFillPath,				(PFN)DrvStrokeAndFillPath},

{INDEX_DrvBitBlt,					(PFN)DrvBitBlt},

{INDEX_DrvCopyBits,					(PFN)DrvCopyBits},

{INDEX_DrvStretchBlt,					(PFN)DrvStretchBlt},

{INDEX_DrvTextOut,					(PFN)DrvTextOut},

{INDEX_DrvEscape,					(PFN)DrvEscape},

{INDEX_DrvDrawEscape,				(PFN)DrvDrawEscape},

{INDEX_DrvQueryFont,					(PFN)DrvQueryFont},

{INDEX_DrvQueryFontTree,				(PNF)DrvQueryFontTree},

{INDEX_DrvQueryFontData,				(PFN)DrvQueryFontData},

{INDEX_DrvSendPage,					(PFN)DrvSendPage},

{INDEX_DrvStartPage,					(PFN)DrvStartPage},

{INDEX_DrvEndDoc,					(PFN)DrvEndDoc},

{INDEX_DrvStartDoc,					(PFN)DrvStartDoc},

{INDEX_DrvGetGlyphMode,				(PFN)DrvGetGlyphMode},

{INDEX_DrvFontManagement,				(PFN)DrvFontManagement},

{INDEX_DrvQueryAdvancedWidths,			(PFN)DrvQueryAdvancedWidths},

{INDEX_DrvStartBanding,				(PFN)DrvStartBanding},

{INDEX_DrvNextBand,					(PFN)DrvNextBand},

};

### 9.3.3 Determination of color or monochrome object

The driver determines whether the object is color or monochrome by looking at a couple of things at DDI calls.  For most DDIs it will be a combination of the items listed below.  The logic will vary for different DDI.  Since the pre-analysis is performed in vector mode, the Vector module must support DrvRealizeBrush.  This also means that it must defined a device brush object for Unidrv5.

DRVREALIZEBRUSH

This function allows GDI to request that the driver creates a device brush based on the psoPattern SURFOBJ passed in.

SURFOBJ.iBitmapFormat specifies the color depth of the pattern brush.  This function is supported by the Vector module.

BRUSHOBJ

BRUSHOBJ.iSolidColor specifies the color index (to the driver created color palette returned to GDI at DrvEnablePDEV time) of the solid brush.   If the brush is not a solid brush, information stored in the driver physical brush structure (initialized at DrvRealizeBrush) will be used to determine the color depth of the brush.

SURFOBJ

For most DDIs, GDI passed in a source surface object.  The SURFOBJ.iBitmapFormat field specifies the format of the object.  Possible values for iBitmapFormat is listed below.

BMF_1BPP	1 bit per pixel

BMF_4BPP	4 bits per pixel

BMF_8BPP	8 bits per pixel

BMF_16BPP	16 bits per pixel

BMF_32BPP	32 bits per pixel

BMF_4RLE	4 bits per pixel, RLE encoded

BMF_8RLE	8 bits per pixel, RLE encoded

DrvBitBlt - Look at the source SURFOBJ.  BRUSHOBJ is only looked at if the rops are SRCCPY and SRCPAINT.

DrvCopyBits - Look at the source SURFOBJ.

DrvStretchBlt -  Look at the source SURFOBJ.

DrvTextOut - Look at the foreground BRUSHOBJ and the opaque BRUSHOBJ.  MIX modes for Unidrv DrvTextOut should always be R2_COPYPEN for both modes.

DrvFillPath  - Look at the source SUROBJ and BRUSHOBJ.

DrvStrokePath - Look at the source SURFOBJ.iBitmapFormat and BRUSHOBJ.

DrvStrokeAndFillPath - Look at the source SURFOBJ.iBitmapFormat and both stroke and fill BRUSHOBJs.

### 9.3.4  Building of linked list of objects’ bounding boxes.

The benefits of building linked list of objects’ bounding boxes are unclear.  Instead of building a linked list of bounding boxes for every object drawn on a page, we could take the union of all objects drawn and use that as our band.  The worst case scenario would be a line drawn on top of the page and a line drawn on the bottom of the page.  In this case, the band would be a whole page.  However, the majority of the documents would not be in this format.  Most document contains objects fairly close together with either the top containing a lot of white space or the bottom containing a lot of white space.  The following illustrates how the band size is calculated.

PUDPDEV->Analysis.rcCurrentDim.top = min (PUDPDEV->Analysis.rcCurrentDim.top, rcObject.top);

PUDPDEV->Analysis.rcCurrentDim.left = min (PUDPDEV->Analysis.rcCurrentDim.left, rcObject.left);

PUDPDEV->Analysis.rcCurrentDim.right = max (PUDPDEV->Analysis.rcCurrentDim.right, rcObject.right);

PUDPDEV->Analysis.rcCurrentDim.bottom =  max(PUDPDEV->Analysis.rcCurrentDim.bottom, rcObject.bottom);

For printers with no Y move command or no line feed command ( PUDPDEV->fMode & PF_NO_Y_MOVE_CMD == 0), optimization for blank spaces cannot be performed since the dump code could not simulate the blank scanlines.  For these printers, the whole page will be processed.

DrvBitBlt

prclDst points to a RECTL structure that defines the area to be modified in the coordinate system of the destination surface.  The rectangle is lower right exclusive.

pco points to a CLIPOBJ structure that limits the area to draw.  pco->rclBounds specifies the clip rectangle to be intersected with prclDst.  pco->iDComplexity specifies the action to be performed with pco->rclBounds.

The Control module will perform the intersection between the clip and destination rectangles as specified by the idComplexity field.

switch (pco->iDComplexity) {

case DC_TRIVIAL:

Do nothing

case DC_RECT:

Check whether target destination rectangle fits inside the clip rectangle.  If so, clip the destination rectangle to pco->rclBounds.

case DC_COMPLEX:

Clip region must be enumerated.

Call CLIPOBJ_ppoGetPath(pco)

Call PATHOBJ_vGetBounds to get the bounding rect of the path.

Clip the destination rectangle to the bounding rect of the path.

}

rcObject is the destination rectangle after any necessary clipping with pco.

DrvCopyBits

prclDst points to a RECTL structure that defines the area to be modified in the coordinate system of the destination surface.  The rectangle is lower right exclusive.  rcObject is prclDst.

pco points to a CLIPOBJ structure that limits the area to draw.  pco->rclBounds specifies the clip rectangle to be intersected with prclDst.  pco->iDComplexity specifies the action to be performed with pco->rclBounds.

The Control module will perform the intersection between the clip and destination rectangles as specified by the idComplexity field.

switch (pco->iDComplexity) {

case DC_TRIVIAL:

Do nothing

case DC_RECT:

Check whether target destination rectangle fits inside the clip rectangle.  If so, clip the destination rectangle to pco->rclBounds.

case DC_COMPLEX:

Clip region must be enumerated.

Call CLIPOBJ_ppoGetPath(pco)

Call PATHOBJ_vGetBounds to get the bounding rect of the path.

Clip the destination rectangle to the bounding rect of the path.

}

rcObject is the destination rectangle after any necessary clipping with pco.

DrvStretchBlt

prclDst points to a RECTL structure that defines the are to be modified in the coordinate system of the destination surface.  However, the coordinates of the second point are not necessarily larger than those of the first points.  In the case where the destination rectangle is not right ordered, the Control module will perform inversions of x and y. The rectangle is lower right exclusive.

pco points to a CLIPOBJ structure that limits the area to draw.  pco->rclBounds specifies the clip rectangle to be intersected with prclDst.  pco->iDComplexity specifies the action to be performed with pco->rclBounds.

The Control module will perform the intersection between the clip and destination rectangles as specified by the idComplexity field.

switch (pco->iDComplexity) {

case DC_TRIVIAL:

Do nothing

case DC_RECT:

Check whether target destination rectangle fits inside the clip rectangle.  If so, clip the destination rectangle to pco->rclBounds.

case DC_COMPLEX:

Clip region must be enumerated.

Call CLIPOBJ_ppoGetPath(pco)

Call PATHOBJ_vGetBounds to get the bounding rect of the path.

Clip the destination rectangle to the bounding rect of the path.

}

rcObject is the destination rectangle after any necessary clipping with pco.

DrvTextOut

pstro points to a STROBJ structure that defines the glyph to be rendered and the positions in which to place them.

pstro->rclBkGround specifies the bounding box for the string.

pco points to a CLIPOBJ structure that limits the area to draw.  pco->rclBounds specifies the clip rectangle to be intersected with prclDst.  pco->iDComplexity specifies the action to be performed with pco->rclBounds.

The Control module will perform the intersection between the clip and destination rectangles as specified by the idComplexity field.

switch (pco->iDComplexity) {

case DC_TRIVIAL:

Do nothing

case DC_RECT:

Check whether target destination rectangle fits inside the clip rectangle.  If so, clip the destination rectangle to pco->rclBounds.

case DC_COMPLEX:

Clip region must be enumerated.

Call CLIPOBJ_ppoGetPath(pco)

Call PATHOBJ_vGetBounds to get the bounding rect of the path.

Clip the destination rectangle to the bounding rect of the path.

}

rcObject is the bounding box for the string after any necessary clipping with pco.

DrvFillPath

ppo points to a PATHOBJ structure.  The driver can call PATHOBJ_vGetBounds(ppo, &rcObject ) to get the bounding rectangle for the paths.

pco points to a CLIPOBJ structure that limits the area to draw.  pco->rclBounds specifies the clip rectangle to be intersected with the rectangle returned from PATHOBJ_vGetBounds.  pco->iDComplexity specifies the action to be performed with pco->rclBounds.  The Control module will perform the intersection between the clip and destination rectangles as specified by the idComplexity field.

switch (pco->iDComplexity) {

case DC_TRIVIAL:

Do nothing

case DC_RECT:

Check whether target destination rectangle fits inside the clip rectangle.  If so, clip the destination rectangle to pco->rclBounds.

case DC_COMPLEX:

Clip region must be enumerated.

Call CLIPOBJ_ppoGetPath(pco)

Call PATHOBJ_vGetBounds to get the bounding rect of the path.

Clip the destination rectangle to the bounding rect of the path.

}

rcObject is the bounding box for the string after any necessary clipping with pco.

DrvStrokePath

ppo points to a PATHOBJ structure.  The driver can call PATHOBJ_vGetBounds(ppo, &rcObject ) to get the bounding rectangle for the paths.

pco points to a CLIPOBJ structure that limits the area to draw.  pco->rclBounds specifies the clip rectangle to be intersected with the rectangle returned from PATHOBJ_vGetBounds.  pco->iDComplexity specifies the action to be performed with pco->rclBounds.  The Control module will perform the intersection between the clip and destination rectangles as specified by the idComplexity field.

switch (pco->iDComplexity) {

case DC_TRIVIAL:

Do nothing

case DC_RECT:

Check whether target destination rectangle fits inside the clip rectangle.  If so, clip the destination rectangle to pco->rclBounds.

case DC_COMPLEX:

Clip region must be enumerated.

Call CLIPOBJ_ppoGetPath(pco)

Call PATHOBJ_vGetBounds to get the bounding rect of the path.

Clip the destination rectangle to the bounding rect of the path.

}

rcObject is the bounding box for the string after any necessary clipping with pco.

DrvStrokeAndFillPath

ppo points to a PATHOBJ structure.  The driver can call PATHOBJ_vGetBounds(ppo, &rcObject ) to get the bounding rectangle for the paths.

pco points to a CLIPOBJ structure that limits the area to draw.  pco->rclBounds specifies the clip rectangle to be intersected with the rectangle returned from PATHOBJ_vGetBounds.  pco->iDComplexity specifies the action to be performed with pco->rclBounds.  The Control module will perform the intersection between the clip and destination rectangles as specified by the idComplexity field.

switch (pco->iDComplexity) {

case DC_TRIVIAL:

Do nothing

case DC_RECT:

Check whether target destination rectangle fits inside the clip rectangle.  If so, clip the destination rectangle to pco->rclBounds.

case DC_COMPLEX:

Clip region must be enumerated.

Call CLIPOBJ_ppoGetPath(pco)

Call PATHOBJ_vGetBounds to get the bounding rect of the path.

Clip the destination rectangle to the bounding rect of the path.

}

rcObject is the bounding box for the string after any necessary clipping with pco.

### 9.3.5  Z-Ordering Detection

9.3.5.1  Summary of Problems

We have two z-ordering problems.  One with device fonts- downloaded TT font and the other with non-banding, vector mode.

Device Fonts and Download TT Font.

In raster mode, we render graphics into our band buffer and send it to the printer.  The printer ORed the band buffer with the texts that have been rasterized on the printer. So we loose the z-ordering.

Non-Banding, Vector Mode

In vector, non-banding mode, there are rops that involve reading from the destination surface (e.g. XOR), then there is really no way to control whether the objects should be appear relative to the destination surface.

9.3.5.2  Possible Solutions

GDI Performs Z-Ordering Detection

To solve this problem, GDI would have to do z-ordering detection.   In banding mode, GDI would notify the driver of the pages in a job where device or TrueType fonts overlaps with non-white graphics.  GDI can proceed to map the device font to a TT font.  The driver, upon notification will not download outline TT font.  It will send bitmap font to the printer.  In addition, it also detects rops that use destination surface.  If the user selected vector mode and these rops are detected, the driver will switch to raster mode and print the job.

Driver Performs Z-Ordering Detection

An alternate solution is to have the driver performs z-ordering detection.  The following  illustrates the determination of z-ordering detection.

PUDPDEV->dwFlags & DETECT_FONT indicates that during the pre-analysis pass, detected device fonts or downloaded TT  fonts overlapping with non-white graphics in opaque mode.  There are two ways to detect device fonts or TT  fonts that overlap with non-white graphics.

The first way is to create two linked list of objects drawn on the page.  One linked list would contain all the text objects drawn with device font or outline TT font on the page.  The other linked list would contain all the non-white graphics objects drawn on the page.  Each node of the linked lists would contain a RECTL that describes the bounding box of the object.  Once the pre-analysis pass concludes, the driver will use the two linked list to check for any overlapping.  This method is very slow and inefficient.

The second way is to create a grid of ½ inch granularity (granularity can varies, it’s inversely proportional to memory requirement to hold the grid and the correctness of the detection).  Every time an object is drawn on the page, it’s marked as text or graphics.  If the same grid is marked again with a different type, we know there is overlapping between text and graphics.  This method is not completely correct.  However, it is faster.

If overlapping is detected the driver would notify GDI to not use any device fonts and print TT as graphics.

PUDPDEV->dwFlags & DETECT_ROPS indicates that during the pre-analysis pass, detected rops that use the destination surface such as XOR.  Note: DETECT_ROPS flag could also be set when the device supports ROPs.  In the case where the device supports ROPs, the Control module will not try to detect rops during pre-analysis pass.  It will just set the DETECT_ROPs flag at initialization.

PUDPDEV->dwFlags & DETECT_OVERRUN indicates that during the pre-analysis pass, detected that the graphics objects count on the page exceeds the limit set in the GPD.  In this case, Unidrv would switch to raster mode in order to avoid print overrun error.

PUDPDEV->dwFlags & DETECT_COMPLEXITY indicates that during the pre-analysis pass, the path objects drawn were too complex and cannot be handled by the device( for example, the number of points on a Bezier curve exceed the number that can be handled by the device).  In this case, Unidrv would switch to raster mode.

### 9.3.6 Real rendering pass

During the real rendering pass, the driver will switch to the printing mode selected, if appropriate.  The following figure illustrates the determination of whether to print in raster mode or vector mode.  The figure assumed that pre-analysis has been performed in the previous playback.

# 10.  OPEN ISSUES

ROP4

PAGE ERROR RECOVERY

ABORTDOC - The spooler will check for AbortDoc on a per page basis (i.e. it will abort the document on a page boundary only).  Once an AbortDoc is detected, the driver must stop sending data to the spooler until it receives EndPage/EndDoc.  It also must get out of graphic state when an AbortDoc occured.

VECTOR vs. RASTER - Should we allow the user to select the print mode?  Or should we let the driver performs the pre-analysis and determine the best print mode?  The problem with this is that pre-analysis is not 100% accurate, it will tend to be on the conservative side, thus more work will need to be done